Project explanation

What is in git_clone/ and how every piece of it works

PE448 Skripsie — External vision system for racetrack mapping and vehicle trajectory tracking

Antwan Gelden · Stellenbosch University · code state as of 28 September 2026 · document updated 5 October 2026

This document describes the code tree ~/Documents/Skripsie/git_clone/ file by file and function by function: what each function takes, what it computes, why it is done that way, and which other functions call it or depend on its result. It is written so that someone who has never opened the code can follow one video from capture/ to the trajectory CSV, the overlay video and the fused two-camera track, and one mapping walk to the dense point cloud, its metric scale and the extracted track boundary.

What changed in this version (5 October 2026). The 22 September version stopped at the dense cloud in SfM units. Since then four files were added to pipeline_B_mapping/ on 28 September: run_boundary.sh, metric_scale.py (contribution 1, metric scale and the shared world frame), boundary.py (contribution 2, the track boundary) and boundary_overlay.py (the visual check). They are described in the new sections 5.5–5.8, and every section that referred to them as "not yet built" has been brought up to date (1, 2.2, 3.4, 4.3, 5, 6 and 7). The only other file touched since 22 September is detector.py, and only its blank lines changed: its behaviour is exactly as section 3.4 describes.

Conventions used here: monospace is a file, function, flag, variable or column name; bold marks a term defined nearby or a number worth remembering. Module names are given relative to git_clone/pipeline_A_tracking/src/ or git_clone/pipeline_B_mapping/src/ unless stated otherwise.

Contents

# 1. What git_clone/ is and where it sits

There are three copies of the tracking code on the machine. Knowing which one you are looking at matters, because a fix in one is not automatically in the others.

| Tree | What it is | State of the tracking code |
| --- | --- | --- |
| ~/Documents/Skripsie/ | The working tree: raw footage in Capture/, the Tracking/ and Mapping/ working folders, context/, V3/, Map_Test/, DOCS/. | Tracking/src/track_ground.py is the 5 September monolith with the old anchor rule (largest sheet). |
| ~/Documents/Skripsie/GIT/ | The clean git repository (github.com/Twan5567/Skripsie, branch skripsie, private). Only the two pipelines, runners and README. Antwan commits and pushes himself. | Modular split of track_ground (detector / geometry / floor / vehicle / outputs) but behind git_clone: old anchor rule, 57 mm tag default, no two-camera code. |
| ~/Documents/Skripsie/git_clone/ | A copy of GIT/ made on 21 September 2026 without a .git folder, used as the workspace for the two-camera build. Nothing is pushed from it. | The newest code. Corrected anchor rule, bounded distortion fit, 175 mm tag default, the whole two-camera pipeline, lens self-calibration, and (28 September) metric scale and boundary extraction for the map. This document describes this tree. |

Until 28 September the mapping side (pipeline_B_mapping/) was identical in GIT/ and git_clone/. It no longer is: run_boundary.sh, metric_scale.py, boundary.py and boundary_overlay.py exist only in git_clone/. Neither the two-camera work nor the boundary work is in the git repository yet.

## 1.1 Folder layout

git_clone/

├── README.md                  user-facing description of both pipelines (302 lines)

├── PLAN_two_camera.md         the approved two-camera plan + "what was built and found"

├── requirements.txt           pycolmap, opencv-contrib, numpy, scipy, open3d, matplotlib (no scikit-learn: 7.2)

├── .gitignore                 capture/* and output/* contents, media, logs, *_work/, .venv

├── pipeline_A_tracking/

│   ├── run_tracking.sh        single-camera runner   ./run_tracking.sh B-5

│   ├── run_dual.sh            two-camera runner      ./run_dual.sh A-5b B-5

│   ├── capture/               symlinks A-1..A-5b, B-1..B-5 -> ../../../Capture/GX01xxxx.MP4

│   │   └── CLIP_MAP.txt       which GoPro file each name is, and the camera position

│   ├── calib/                 camA.json, camB.json, camB_fixed.json (+ .log) from calibrate_lens.py

│   ├── output/                one folder per run: A-1_B-1/ ... A-5b_B-5/

│   └── src/

│       ├── track_ground.py    single-camera driver: process_clip() and the CLI

│       ├── track_dual.py      two-camera driver

│       ├── detector.py        tuned ArUco detector, pass 1 over the video, windowed search

│       ├── geometry.py        undistort / distort, square error, camera from homography

│       ├── floor.py           floor-plane fit from the GCP sheets  -> FloorFit

│       ├── vehicle.py         vehicle fixes, parallax, blob fallback, smoothing -> Track

│       ├── outputs.py         CSV, (plots), overlay video, summary  (single camera)

│       ├── timecode.py        tmcd timecode -> global seconds of day

│       ├── world.py           GCP-defined world frame, 2-D rigid fit A<->B -> WorldFit

│       ├── fuse.py            common time grid, merge, Kalman/RTS, tag-height solve -> Fused

│       ├── crossguide.py      each camera searches where the other saw the car

│       ├── dual_outputs.py    fused CSV, top-down PNG, side-by-side overlay

│       ├── calibrate_lens.py  pooled per-camera-body lens fit -> calib/*.json

│       ├── aruco_config.py    the ArUco dictionary (DICT_4X4_50)

│       └── gpu.py             OpenCV OpenCL initialisation

└── pipeline_B_mapping/

├── lib.sh                 shared bash helpers for the four runners

├── run_mapping.sh         frames -> COLMAP sparse -> (dense)

├── run_dense.sh           dense only, for an existing sparse model

├── run_splat.sh           OpenSplat Gaussian splat, with a memory guard

├── run_boundary.sh        metric scale -> boundary -> overlay video   ./run_boundary.sh TrackMap_S2

├── capture/               TrackMap1_4k.MP4, TrackMap2_4k.MP4 -> ../../../Capture/GX011488/1489.MP4

├── output/TrackMap_S2/    sparse/ (copied), _dense.ply (hard link), metric + boundary outputs

└── src/

├── extract_frames.py       ffmpeg -> PNG at a fixed fps

├── colmap_pipeline.py      feature_extractor -> matcher -> mapper

├── dense_reconstruction.py colmap undistort -> OpenMVS (Docker)

├── plot_topdown.py         RANSAC floor plane + top-down render (sparse health check)

├── metric_scale.py         GCP sheets -> scale, floor plane, world frame  -> <NAME>_metric.json

├── boundary.py             metric dense cloud -> wall centrelines, midline, width, DXF

└── boundary_overlay.py     boundary projected back into the mapping frames -> MP4

## 1.2 Conventions shared by both pipelines

- Videos go in capture/, runs are by name, results land in output/<name>/. You never pass a path to a runner; you pass A-5b, and the runner looks for capture/A-5b.MP4 (any case of .mp4/.mov).

- Every runner finds the Python interpreter the same way: $PYTHON if set, else ../.venv/bin/python or ../../.venv/bin/python, else python3.

- Everything a run prints is also written to output/<name>/<name>.log through tee.

- Both pipelines rely on ArUco sheets from DICT_4X4_50 lying flat on the floor: IDs 0–11 are ground control points (GCP, 150 mm black square), 20 and 21 are the vehicle tags (20 = the one actually used in session 2), 30–31 scale-bar endpoints. Only sheets 0–5 were laid in session 2.

- Flags after -- on a runner's command line are passed straight through to the Python script, so any CLI option can be overridden without editing the runner.

## 1.3 The footage the code was built on

Session 2 (11 September 2026): two GoPro HERO13 Black bodies at opposite ends of an indoor track, 3840×2160 @ 59.94 fps HEVC, both scanned a GoPro QR time-sync code so their embedded tmcd timecode is global time (verified by audio cross-correlation to within 47 ms). capture/CLIP_MAP.txt maps the symlink names to files:

| Name | GoPro file | Camera position | Notes |
| --- | --- | --- | --- |
| A-1 / B-1 | GX010165 / GX011480 | A1 / B1 | pair 1, ~85 s; A-1 tracked from 23 s (--start-a 23) |
| A-2 / B-2 | GX010166 / GX011482 | A1 / B1 | pair 2 |
| A-3 / B-3 | GX010167 / GX011483 | A1 / B1 | pair 3 |
| A-4 / B-4 | GX010168 / GX011484 | A2 / B1 | pair 4; camera A moved before this take |
| A-5 / B-5 | GX010169 / GX011487 | A2 / B2 | pair 5; cam A left rolling, 16 s overlap |
| A-5b / B-5 | GX010170 / GX011487 | A2 / B2 | pair 5 alt, 74 s overlap — the best pair to look at |

The N in A-N/B-N is the pair number, not the camera position. Both cameras moved once mid-session (A1→A2, B1→B2), so each position needs its own floor fit, which the code does per clip anyway.

The mapping side uses the two hand-held walks of the same session, camera B, filmed while the floor sheets were down: GX011488 and GX011489, linked in pipeline_B_mapping/capture/ as TrackMap1_4k.MP4 and TrackMap2_4k.MP4. Combined at 4 fps they are one frame set of 577 frames (prefixes w1_/w2_), which COLMAP registered 577 / 577 in one component with 66,439 sparse points (mean reprojection error 1.06 px) and OpenMVS densified to 16.2 M points. That model, TrackMap_S2, was built on 15 September in the working tree (Mapping/output/TrackMap_S2/); on 28 September its sparse/ folder was copied into git_clone/pipeline_B_mapping/output/TrackMap_S2/ and its dense PLY hard-linked there, and the boundary stage was run on it.

# 2. Pipeline A, single camera — how one clip becomes a trajectory

The idea in one paragraph: a fixed camera looks down at the track. ArUco sheets of known size (150 mm) lie flat on the floor. Because the car drives on the floor, everything of interest is in one plane, and a plane seen by a pinhole camera is related to the image by a homography (a 3×3 matrix). The sheets give that homography: their corners are at known relative positions in the world (a square of side 150 mm), and the detector finds where they are in the image. Once the homography is known, any pixel on the floor can be turned into millimetres. The vehicle carries its own tag; where the detector finds that tag, its pixel centre goes through the same homography (after two corrections: lens distortion and the fact that the tag sits 175 mm above the floor) and out comes the car's floor position in millimetres. Do that for every frame and you have a trajectory.

Three things make the real version more work than that paragraph: a GoPro lens is far from a pinhole (so radial distortion must be estimated from the sheets themselves), a homography built from one sheet is noisy and can be badly conditioned (so the anchor sheet is chosen carefully and the fit is refined on all sheets), and the vehicle tag is small and oblique so it is missed in some frames (so there is a prediction-gated second look, and smoothing).

## 2.1 Order of operations (call graph)

run_tracking.sh  B-5

└─ python track_ground.py --video capture/B-5.MP4 --outdir output --stride 2 --smooth 15 --tag-heights 20:175 --no-fallback

main()

└─ process_clip(video, outdir, args)

├─ detector.build_detector()                  tuned cv2.aruco.ArucoDetector

├─ detector.detect_all_frames(...)  -> Detections   PASS 1: every marker, every 2nd frame

│    └─ _VehiclePredictor + _window_search        second look for the car tag

├─ timecode.clip_start_seconds() + frame_times()   global time per analysed frame

├─ floor.fit_floor(...)           -> FloorFit      which sheets are static, distortion,

│    ├─ geometry.undistort / square_error            anchor choice, homography H,

│    └─ geometry.camera_from_homography              focal length, camera height, nadir

├─ vehicle.locate_vehicle_aruco(...)                 tag centre -> floor mm + parallax

├─ vehicle.blob_fallback(...)        (only without --no-fallback)

├─ vehicle.combine_and_smooth(...) -> Track          merge, origin shift, Savitzky-Golay

└─ write_clip_outputs(res, args)

├─ outputs.write_csv        <stem>_trajectory.csv

├─ outputs.write_plots      (disabled)

├─ outputs.write_overlay    <stem>_overlay.mp4   PASS 2 over the video

└─ outputs.print_summary

Two full reads of the video happen: pass 1 for detection, pass 2 to draw the overlay. The blob fallback (off by default) would add two more reads (one to build the background, one to subtract it).

## 2.2 Coordinate frames — read this before the function descriptions

Five different 2-D coordinate systems appear in the tracking code. Confusing them is the easiest way to misread a number, so here they are in the order the data passes through them:

- Raw pixels — what the detector returns, in the 3840×2160 frame as recorded, lens distortion and all. Detections.corners_by_frame, FloorFit.med, Track.px, the px_x/px_y and aruco_px_x/y CSV columns are in raw pixels.

- Undistorted pixels — raw pixels after geometry.undistort() with the fitted k1, k2. FloorFit.med_u is here. The homography FloorFit.H maps from this frame.

- Camera floor frame (anchor frame), mm — the output of H. Origin is the first corner of the anchor sheet, axes are that sheet's edges, unit is millimetres. FloorFit.nadir is in this frame; Track.mm_raw + Track.origin returns to it.

- Track frame, mm — the anchor frame shifted so that the first located frame of the car is (0, 0). Track.mm and the mm_x/mm_y CSV columns are here. Only the origin differs from frame 3; Track.origin holds the shift.

- World frame, mm (two-camera only) — defined by the sheet layout, not by any camera: origin at the centre of GCP 1, +x toward the centre of GCP 5, y perpendicular with the same handedness. WorldFit, Fused.xy, the _fused.csv and the top-down PNG are here. Since 28 September the map is brought into this same frame by metric_scale.py (5.5), using the same two sheets, so a fused trajectory and the track boundary can be drawn on one plot without any survey.

Handedness note: ArUco corner order is top-left, top-right, bottom-right, bottom-left in the image, and image y runs downward, so the anchor frame is left-handed relative to a z-up world. camera_from_homography() compensates by taking the absolute value of the recovered height; nothing else needs to care because every frame in the chain has the same handedness.

The world frame is mirrored. The same left-handedness carries into the world frame: seen from above with z up, the tracker's +y points the opposite way from the usual x-right, y-up picture, so the determinant of the map from a true top view to (x, y) is −1. metric_scale.py copies this on purpose so the map and the trajectories share one frame (it prints tracker (x, y) seen from above is MIRRORED (det R = -1)). Consequence: every top-down PNG either pipeline writes is a mirror image of the real track. Lengths, areas, widths and distances are unaffected. If the frame is ever flipped, flip y in both pipelines together, or the trajectories will no longer sit inside the map.

# 3. Pipeline A, single camera — module by module

## 3.1 run_tracking.sh — the single-camera runner

A 75-line bash script. Its job is to turn names into paths, bake in the defaults, and log everything.

- Arguments: zero or more clip names; --fallback to re-enable the blob fallback; everything after -- is passed to Python untouched. With no names it runs every .mp4/.mov in capture/.

- For each name it tries capture/<name>.MP4, .mp4, .MOV, .mov and exits with a clear message if none exists.

- It then runs, from inside src/: track_ground.py --video <clip> --outdir ../output --stride 2 --smooth 15 --tag-heights 20:175 --no-fallback <extra>, piping stdout+stderr through tee into output/<stem>/<stem>.log. PIPESTATUS[0] is used so the Python exit code, not tee's, is reported.

- Baked-in defaults and why: --stride 2 analyses every second frame (30 Hz effective; the car does not move enough in 33 ms to need more and it halves the runtime). --smooth 15 is the Savitzky-Golay window in analysed frames (half a second). --tag-heights 20:175 is the session-2 tag on the roof of the ~175 mm tall car — a looked-up value, still to be measured. --no-fallback because with 83–91 % detection the blob fallback is unnecessary and was found to corrupt neighbouring ArUco frames through the smoothing window.

## 3.2 aruco_config.py — the dictionary

Holds ARUCO_DICT = cv2.aruco.getPredefinedDictionary(cv2.aruco.DICT_4X4_50). Every other module imports the dictionary from here so it is defined once. The file also carries constants from earlier phases (TEST_MARKER_SIDE_MM = 100, FULL_MARKER_SIDE_MM = 250, A4_SHEET_SIDE_MM = 184.57) which the current pipeline does not use — the sheet side is a CLI flag, --gcp-mm, defaulting to 150. Only ARUCO_DICT matters now.

## 3.3 gpu.py — OpenCL initialisation

init_opencv_opencl(enable=True) -> bool

Sets RUSTICL_ENABLE=radeonsi in the environment (the Mesa OpenCL driver for the AMD card), imports cv2, and if OpenCL is available switches OpenCV's transparent API on with cv2.ocl.setUseOpenCL(enable). Returns whether it is now active.

It is called as the first line of track_ground.py, track_dual.py and calibrate_lens.py, before any other import, because the environment variable must be set before cv2 is first imported. In practice the ArUco detector and colour conversion run on the CPU regardless; this is a harmless best-effort speed-up.

report() -> str

Returns a short human-readable status of OpenCV OpenCL and the known CUDA-only limitations. Only used when the module is run directly (python gpu.py).

## 3.4 detector.py — finding the markers

This module owns everything about ArUco detection: the tuned detector parameters, the first pass over the video that records every marker in every analysed frame, the vehicle-tag predictor, and the windowed, upscaled second look that recovers small tags.

build_detector() -> cv2.aruco.ArucoDetector

Creates a detector on ARUCO_DICT with parameters tuned for small, oblique markers in a 4K frame. The important one is minMarkerPerimeterRate = 0.008: OpenCV's default is 0.03 and is relative to the larger image dimension (a 115 px minimum contour perimeter at 3840 px); 0.008 lowers that to 31 px so the small, far sheets and the ~35 px vehicle tag at the far end of a view are never rejected on size before decoding is even attempted. The adaptive threshold windows are set to 3–45 px in steps of 4 so that both large near sheets and small far ones get a suitable window; polygonalApproxAccuracyRate = 0.06 tolerates the curved edges a wide lens produces; corner refinement is CORNER_REFINE_SUBPIX because sub-pixel corners are what the homography fit needs.

Called by process_clip(), by crossguide.guide(), by calibrate_lens.sample_poses() and, since 28 September, by pipeline B's metric_scale._detect(), which imports it across the tree so that the map finds the floor sheets with exactly the same tuned parameters as the tracker.

parse_ids(spec: str) -> list[int]

Turns a spec like "0-11,20" into [0,1,...,11,20]. Used for --gcp-ids and --vehicle-ids.

class Detections  (dataclass)

What pass 1 learns about a clip. corners_by_frame is a list with one entry per analysed frame; each entry is a dict marker id -> 4×2 array of raw-pixel corners. fps, W, Hh are the video properties. frame_i is the total number of frames decoded, n the number analysed (frame_i / stride, rounded up). recovered is the set of analysed-frame indices where the vehicle tag came from the windowed second look rather than the full-frame pass, and rejected counts vehicle-ID decodes thrown away by the prediction gate.

This object is carried inside ClipResult and is mutated in place later by crossguide.guide(), which adds tag corners it found into corners_by_frame.

class _VehiclePredictor

A tiny constant-velocity predictor for the car in pixels, fed by the last two fixes. It exists for two reasons: to aim the windowed second look when the full-frame pass misses the tag, and to reject a vehicle-ID decode that appears somewhere the car cannot be (a misread sheet, a reflection).

update(corners) sets pos to the tag centre, vel to the displacement since the previous fix divided by its age (only if the previous fix is at most 3 frames old, otherwise velocity resets to zero), side to the mean side length in pixels, and age to 0. tick() increments age once per analysed frame. predict() returns pos + vel × min(age, 3), or None if there was never a fix or the last one is older than memory (30 analysed frames = 1 s): the velocity extrapolation is capped at three frames so a long coast does not fly off the track. gate_px() returns the radius within which a decode is accepted: max_step_px × min(age, 3) + 2 × side — a car cannot move more than --max-step-px (260 px) per analysed frame, plus a margin of two tag widths.

_window_search(gray, detector, pred, side, gate, veh_ids, target_px=110) -> (id, corners) | None

The second look. Crops a square window around the predicted position pred with half-size max(gate, 4 × side, 160) px, upscales it with cubic interpolation so the tag would come out about target_px (110) pixels wide (scale clamped to 1–4×), and runs the detector on the crop. A 35 px tag at the far end of the view has about 6 px per code cell, right at the decoder's limit; upscaling gives the adaptive threshold and the cell sampling something to work with. Any vehicle-ID decode is mapped back to full-frame pixels (divide by scale, add the crop offset) and accepted only if its centre is within gate px of the prediction; the nearest one wins.

Called from detect_all_frames() (guided by the predictor) and from search_frames() (guided by the other camera).

detect_all_frames(video, stride, detector, veh_ids=None, max_step_px=260) -> Detections

Pass 1. Opens the video with cv2.VideoCapture, reads every frame, and for every stride-th frame converts to grey and runs detector.detectMarkers(). All decoded markers go into a dict id -> corners for that frame.

If veh_ids is given (it is, unless --no-window-search), the predictor is ticked and consulted every analysed frame: (1) any vehicle-ID decode further than gate_px() from the prediction is deleted and counted in rejected; (2) if no vehicle tag survived and a prediction exists, _window_search() is tried and a hit is inserted into the frame's dict and its index added to recovered; (3) the predictor is updated with whichever vehicle tag is present. Note that step 1 only applies when a prediction exists, so the first fix of a clip is always accepted.

Prints a progress counter every 200 frames and a final line with the number of frames analysed, the number recovered by the windowed search and the number rejected. Raises SystemExit if the video cannot be opened or no frames decoded.

Cost: this pass dominates runtime (~1 min for a 70 s clip at stride 2). The full-frame ArUco detection on a 4K grey image is the expensive part.

search_frames(video, stride, detector, targets, veh_ids, gate) -> dict[idx -> (id, corners)]

A targeted windowed search used by cross-camera guidance. targets maps analysed-frame index → (predicted tag centre in this camera's pixels, expected tag side in px). The clip is read once sequentially with cap.grab() (cheap: no decode) and only the frames in targets are retrieved and searched with _window_search(). Returns the hits. Stops reading after the last requested frame.

## 3.5 geometry.py — pure functions

Nothing in this module reads a video or prints; every function maps arrays to arrays. It is imported by floor.py, vehicle.py, outputs.py, world.py, crossguide.py, dual_outputs.py and calibrate_lens.py.

undistort(pts, c, s, k1, k2) -> pts

Radial undistortion with the division model about the image centre c. For a point at normalised radius r = |p − c| / s, the undistorted point is c + (p − c) / (1 + k1·r² + k2·r⁴). s is a radius normaliser (half the image diagonal, 2203 px for 4K) so that k1, k2 stay of order one.

The division model is used rather than OpenCV's polynomial model because it works in pure image space and needs no focal length — important because the focal length is itself recovered later from the undistorted homography. Negative k1 means barrel distortion is being removed (camera B: k1 ≈ −0.10 to −0.17); positive means the opposite (camera A pins at +0.5, which is one of the signs that camera A's images do not fit the model).

distort(pts, c, s, k1, k2, iters=12) -> pts

The inverse of undistort: takes undistorted pixels back to where the lens would have put them. The division model has no closed-form inverse, so it iterates d_d = d_u × (1 + k1·r_d² + k2·r_d⁴) twelve times on the distorted radius, which converges quickly for lens-sized coefficients. Used only for drawing: the fused world track has to be reprojected into the raw video for the overlay (dual_outputs._world_to_px) and for aiming the cross-camera search (crossguide._px_of_world).

square_error(med_u, anchor, S) -> (rms_mm, per_sheet_mean_side, H_anchor)

The cost function of the whole floor fit. Builds a homography from the anchor sheet's four (undistorted) corners to the ideal square [(0,0),(S,0),(S,S),(0,S)], pushes every sheet's corners through it, and measures how far each sheet's four sides are from S and its two diagonals from S√2. Returns the RMS of all those errors in mm, a dict of each sheet's mean side length, and the homography.

Because the anchor is pinned to S by construction, this number measures how well the rest of the floor agrees with the anchor — which is exactly what lens distortion and a bad anchor break. Diagonals are included because they are scale-aware in a different way from the sides and prevent a uniform-shrink degeneracy. Returns 1e9 if findHomography fails.

camera_from_homography(Hf2i, c) -> (f_px, camera_xyz_mm, (f_a, f_b))

Recovers the focal length and the camera position from one floor→image homography Hf2i (the inverse of FloorFit.H). It assumes square pixels and the principal point at the image centre c, which leaves the focal length f as the only unknown in the intrinsic matrix. Zhang's two orthonormality constraints on the first two columns of the rotation (they must be orthogonal, and of equal length) each give an independent estimate f_a, f_b; the returned f is the RMS of the valid ones and the pair is returned too, because their agreement is a free check on the pinhole assumption (camera B: 1798 / 1831 px; when they disagreed by 26 % in September that was the lens talking).

With f known, K⁻¹ · H gives [r1 r2 t] up to scale; the scale λ is fixed by |r1| = 1, r3 = r1 × r2, the result is snapped to the nearest true rotation by SVD, and the camera centre is C = −Rᵀ t in floor-frame millimetres. The z component is returned as |C_z| because the anchor frame is left-handed (see 2.2). C[:2] is the nadir — the point on the floor directly below the camera — and C[2] is the camera height. The recovered height against the tape-measured one (3271 mm in session 1) is the pipeline's self-check: 3099 / 3275 / 3221 mm on Mono1–3 with the corrected anchor rule.

apply_H(Hm, pts) -> pts

Thin wrapper around cv2.perspectiveTransform that accepts any point-array shape and returns the same shape. Used everywhere a homography is applied.

## 3.6 floor.py — the floor-plane fit

One big function, fit_floor(), that goes from Detections.corners_by_frame to a FloorFit. It is the metric heart of the pipeline: everything downstream inherits its scale and its errors. The logic is described here step by step because each step exists because of a specific failure.

class FloorFit  (dataclass)

rates: id → fraction of analysed frames in which the marker was decoded (all ids, not just GCPs). static: the subset accepted as floor GCPs, with their rates. med: id → median 4×2 raw-pixel corners over the clip; med_u: the same after undistortion. anchor: the chosen anchor sheet id. S: the sheet side in mm (150). img_c, norm_s: distortion-model centre and normaliser. k1, k2: the fitted distortion. H: the 3×3 homography undistorted pixels → anchor-frame mm. f_px: recovered focal length. nadir: (x, y) on the floor below the camera, anchor frame. H_cam: recovered camera height in mm. resid: per-sheet side-length residual (mean side − S) in mm; per_side: id → mean side length.

fit_floor(corners_by_frame, n, W, Hh, gcp_ids, veh_ids, min_rate, anchor_id, gcp_mm, no_distortion, cam_height_mm, lens=None) -> FloorFit

Step 1 — which markers are static floor sheets. rates[k] is computed for every id seen. spread_px(k) is the 90th percentile of the distance of each detection's centre from the median centre. A sheet is accepted into static if its id is in gcp_ids, its rate is at least min_rate (0.60) and its spread is under 5 px: a glued-down sheet must barely move in the image, and this is what separates a real GCP from a spurious decode of floor tiling. The 90th percentile rather than the maximum is deliberate — one misread of the car's tag as a sheet ID, 1000 px away, must not disqualify a sheet that sat still for every other frame (this happened on session-2 footage with ID 5).

Step 2 — median corners. med[k] is the per-corner median over all frames where the sheet was decoded. Median because it is robust to the occasional bad decode; the corner refinement noise is then averaged down over hundreds of frames.

Step 3 — the distortion fit, per candidate anchor. fit_distortion(anc) minimises square_error(undistort(med, k1, k2), anc, S) over (k1, k2) with Nelder-Mead from three starting points (0, −0.1, +0.1). Every sheet is a known square, so the distortion that makes all of them square at once can be solved for directly — no ChArUco board required for this job. Two guards: k2 is only fitted when there are at least 5 sheets (fit_k2), because with four sheets clustered near the centre the two-parameter model is under-determined and once produced k1 = +1.8, k2 = −6.7 — a model that goes negative before the image edge and reflects everything out there; and any candidate with |k1| > 0.5, |k2| > 0.25, or a division factor outside 0.5–1.5 anywhere in the frame is rejected with a 1e6 penalty. If a lens file is given the coefficients are taken from it and only the residual is computed; with --no-distortion or fewer than 3 sheets, k1 = k2 = 0.

Step 4 — the anchor choice (the 21 September fix). Unless --anchor-id is forced, every static sheet with rate > 0.9 is tried as the anchor, each with its own distortion fit, and the one whose fit gives the lowest residual on the other sheets wins. The old rule (largest sheet in pixels) picks the most foreshortened sheet from a low, oblique camera, and a homography from four nearly-collinear corners is so ill-conditioned that sub-pixel noise throws the far sheets to tens of metres: from position A2 the largest sheet (ID 0) gave a 2400 mm residual where ID 1 gave 13 mm. On the overhead mono clips the same change took the scale error from ~5 % to 1.3–2.6 %. The line anchor candidates : 0: 2400.5 mm, 1: 13.1 mm, 5: 17.3 mm -> 1 in every log is this step.

Step 5 — the anchor homography and its refinement. H0 = findHomography(med_u[anchor], square) maps the anchor's four corners to the ideal 150 mm square. That rests on one marker's four corners, so its noise is that marker's noise. Rectifying every other sheet through H0 gives their floor layout; refitting a homography from all undistorted corners to that layout with RANSAC (2 px threshold) averages the corner noise down without changing the scale, which still comes from the anchor alone. That refined matrix is H.

Step 6 — camera pose. camera_from_homography(inv(H), img_c) gives focal length, nadir and height. If --cam-height-mm was passed the difference is printed as a percentage; the flag changes nothing else.

Step 7 — the report. Prints the distortion coefficients with the RMS before/after, the focal length and its two estimates, the height, the nadir, then a table of every accepted sheet (rate, pixel side, recovered mm side, error) and, for any marker decoded in more than half the frames that is not in the GCP set, its implied physical size — useful for spotting a mislabelled sheet. Ends with the side-length residual mean and RMS over the sheets; that RMS divided by 150 is the scale check percentage in the summary.

Calls: geometry.undistort, geometry.square_error, geometry.apply_H, geometry.camera_from_homography, cv2.findHomography, scipy.optimize.minimize. Called by track_ground.process_clip() once per clip.

## 3.7 vehicle.py — locating the car

smooth_xy(arr, ok, win, poly=2) -> arr

Savitzky-Golay smoothing of an (n, 2) array, applied per contiguous run of located samples (ok mask). The runs are found with np.split on breaks in the index sequence; runs shorter than 5 are left alone; the window is shrunk to an odd number no larger than the run. Filtering straight through a dropout would drag the fit across the gap and invent a shortcut, so each unbroken run is smoothed on its own. Savitzky-Golay rather than a moving average because it fits a local quadratic: it takes the jitter out of a corner without rounding the corner off. Returns the input unchanged if win < 5.

Used by combine_and_smooth() (single camera) and by fuse.fuse() when --fusion mean is chosen.

locate_vehicle_aruco(corners_by_frame, n, veh_ids, floor, tag_h) -> (veh_px, veh_mm, veh_src, aruco_hits)

For every analysed frame, takes the first vehicle id present (20 before 21), its corner centre in raw pixels, undistorts it, applies floor.H to get a floor point, and then applies the parallax correction: a tag h above the floor is imaged as if it were further from the nadir by a factor H_cam / (H_cam − h), so the corrected floor point is nadir + (raw − nadir) × (H_cam − h) / H_cam. With the camera height known from the homography this correction is exact and needs no calibration. At 4 m from the nadir with h = 175 mm and H_cam = 3200 mm the shift is about 220 mm — using the wrong h (57 mm, as every run before 21 September did) costs ~150 mm at the far side of the track.

Returns arrays of length n: veh_px (raw pixel centre, NaN where no tag), veh_mm (corrected anchor-frame mm), veh_src ('aruco20' etc. or ''), and the count of frames with a fix. Prints the mean and maximum parallax shift.

blob_fallback(video, stride, frame_i, n, W, Hh, veh_px, max_step_px) -> blob_px

Off by default (--no-fallback). Median-background subtraction to fill frames where the tag did not fire. Pass 1: reads 40 frames spread evenly across the clip (BG_N), downscaled to 480 px wide (BG_W), and takes the per-pixel median — the empty floor, since the car occupies any given pixel for a small fraction of the run. Pass 2: for every analysed frame, absdiff against the background, threshold at 28, morphological open then close, external contours, keep those with area 30–4000 px² at the reduced scale. The blob nearest the previous position is chosen (seeded on the first real ArUco fix, not on whatever is biggest in frame 0), and a blob that teleports further than max_step_px is dropped so the track coasts rather than jumping to a person walking past. Returns positions scaled back to full-frame pixels, NaN where nothing was accepted.

Why it is off: at 83–91 % ArUco detection it is unnecessary, and one bad blob fill (e.g. latching onto a person beside the track, as happened on Mono3) drags the ~7 ArUco frames either side of it through the Savitzky-Golay window. The px and aruco_px CSV columns let you see this: px is smoothed+merged, aruco_px is the raw detection.

class Track  (dataclass)

px, mm: the smoothed, merged pixel and floor tracks (mm in the track frame, origin = first located frame). origin: the anchor-frame position of that first located frame, so mm + origin is back in the anchor frame — this is what the two-camera code uses. px_raw, mm_raw: before smoothing. have: located mask. fill: frames filled by the blob fallback. veh_src: per-frame source string. dist: distance from start, NaN where not located. raw_path, path, net: path lengths (raw is jitter-inflated) and net displacement, in mm.

combine_and_smooth(veh_px, veh_mm, veh_src, blob_px, floor, tag_h, smooth) -> Track

Merges: wherever the ArUco position is NaN and a blob exists, the blob pixel is taken, converted through undistort → H → parallax (with the mean tag height, since the silhouette centroid sits on the body, not on the floor), and marked 'blob'. Raises if the vehicle was never located. Shifts the origin to the first located frame. Keeps raw copies, smooths mm and px with smooth_xy, computes path lengths and the distance-from-start series for the overlay HUD. The raw path length is reported alongside the smoothed one because detector jitter accumulates as distance — a car sitting still racks up path length — and the ratio is a useful sanity number.

## 3.8 outputs.py — writing the single-camera results

write_csv(path, track, veh_px, stride, fps, t_global=None)

One row per analysed frame: frame (source frame number = i × stride), time_s (frame / fps), time_global_s (seconds of day from the timecode, empty if the clip has none), source (aruco20 / aruco21 / blob / empty), px_x, px_y (smoothed merged raw pixels), mm_x, mm_y (track frame mm), aruco_px_x, aruco_px_y (the raw detection, empty where none). The last two columns are the ground truth for whether a frame was really measured.

write_plots(outdir, stem, track, floor, stride, fps, smooth)

Disabled. The body is the original trajectory / displacement plotting code, commented out pending Antwan's rework; the function currently only binds the variable names it will need. _trajectory.png and _distance.png are therefore not written by the single-camera run.

write_overlay(video, out_path, track, floor, stride, fps, W, Hh, overlay_width)

Pass 2 over the video. Reads every frame, keeps every stride-th, resizes it to overlay_width (1920) wide, and draws: each static GCP's median outline in grey with its id; the smoothed pixel track so far as a yellow polyline (px[:k+1][have[:k+1]] — note this draws a single polyline through all located frames so far, i.e. a chord across any dropout; the two-camera overlay fixes that, this one has not been changed); the current position as a filled circle, red for an ArUco fix and green for a blob fill; and a two-line HUD with frame number, time, source and distance from start. Written with the mp4v codec at fps / stride so it plays at real speed. This video is how Antwan judges a run — never skip it.

print_summary(stem, W, Hh, fps, frame_i, n, stride, floor, track, aruco_hits, smooth)

The block at the end of every log: resolution and frame counts, GCPs used and anchor, vehicle ArUco fixes and percentage, blob-filled frames, located total, raw and smoothed path length, net displacement, and the scale check — RMS of the sheet side residual in mm and as a percentage of 150 mm.

## 3.9 timecode.py — global time

clip_start_seconds(video, fps) -> float | None

Runs ffprobe -show_entries stream_tags=timecode on the clip and parses the tmcd string HH:MM:SS:FF (drop-frame ; is normalised to :) into seconds of day: h·3600 + m·60 + s + f / round(fps). The frames field counts whole frames at the nominal rate (60 for 59.94). Returns None if ffprobe fails or there is no timecode, in which case the single-camera CSV simply has an empty time_global_s column and two-camera fusion refuses to run.

frame_times(start, n, stride, fps) -> ndarray | None

start + arange(n) × stride / fps: the global time of every analysed frame.

fmt_tod(t) -> str

Formats seconds of day as HH:MM:SS.mmm for the logs and the overlay HUD.

## 3.10 track_ground.py — the single-camera driver

The docstring says it: this file is only the command line and the order of operations. After the 21 September refactor its body is a function, process_clip(), so that track_dual.py can call it once per camera.

add_tracking_args(ap)

Registers every flag except --video/--outdir on an ArgumentParser, so track_dual.py gets exactly the same options. The flags are in the table below.

| Flag | Default | Meaning |
| --- | --- | --- |
| --gcp-ids | 0-11 | IDs of the flat floor sheets |
| --gcp-mm | 150 | black-square side of the floor sheets, mm (measure the print) |
| --vehicle-ids | 20,21 | vehicle tag IDs, in order of preference |
| --anchor-id | auto | force the anchor sheet instead of choosing by squareness |
| --min-rate | 0.60 | detection rate a sheet needs to be a floor GCP |
| --stride | 1 (runner: 2) | analyse every Nth frame |
| --no-fallback | off (runner: on) | skip the blob fallback |
| --cam-height-mm | none | measured camera height; only changes what is printed |
| --tag-heights | 20:57,21:67 (runner: 20:175) | height of each vehicle tag above the floor, mm |
| --lens | none | lens JSON from calibrate_lens.py; fixes k1, k2 and the centre |
| --no-distortion | off | leave k1 = k2 = 0 |
| --max-step-px | 260 | largest plausible car motion per analysed frame, full-res px |
| --smooth | 15 | Savitzky-Golay window in samples; 0 disables |
| --no-window-search | off | disable the predictor-guided second look |
| --start-s | 0 | ignore vehicle fixes before this time (floor fit still uses the whole clip) |
| --no-overlay | off | skip the overlay video |
| --overlay-width | 1920 | overlay width in px |

parse_tag_heights(spec) -> dict[int, float]

"20:175,21:67" → {20: 175.0, 21: 67.0}.

class ClipResult  (dataclass)

Everything one single-camera run produced, for a second stage to build on: stem, video, outdir (already <outdir>/<stem>), stride, det: Detections, floor: FloorFit, track: Track, veh_px (raw tag centres), aruco_hits, t_global. track_dual.py holds two of these.

process_clip(video, outdir, args, write_outputs=True) -> ClipResult

The complete single-camera run, in the order shown in 2.1: create <outdir>/<stem>/, parse the id and tag-height specs, detect_all_frames (with the predictor unless --no-window-search), timecode, optional lens JSON, fit_floor, locate_vehicle_aruco, blob_fallback or an all-NaN array, then — if --start-s > 0 — blank every fix before that time (frames are kept so numbering holds; this is how A-1's first 23 s of the car being placed are ignored), combine_and_smooth, and finally write_clip_outputs unless the caller wants to improve the fixes first (the two-camera driver passes write_outputs=False).

The refactor was verified by checking that the Mono1 CSV came out byte-identical to the monolith's before any behavioural change was made.

relocate_vehicle(res, args)

Re-runs only the vehicle stage of a ClipResult after det.corners_by_frame gained new tag fixes from cross-camera guidance: locate_vehicle_aruco again, the same --start-s blanking, and combine_and_smooth. The floor fit is untouched (static sheets only) and the blob fallback is not re-run — the previous blob fills are re-used only where they were used before, and only if the fallback was on.

write_clip_outputs(res, args)

CSV, (plots), overlay and summary for one ClipResult, into res.outdir.

main()

parse_args() then process_clip(args.video, args.outdir, args).

# 4. Pipeline A, two cameras — fusing the pair

Session 2 put a second camera at the other end of the track. Each clip alone gives a metric floor-plane track through process_clip(); the two-camera code answers three questions: how do the two cameras' floor frames relate (world.py), how do the two tracks line up in time and merge (fuse.py), and can each camera help the other find the tag (crossguide.py). The choice was to fuse in the floor plane, not by triangulating rays in 3-D: triangulation needs each camera's intrinsics and full pose, and the focal length is only recovered from the homography (uncalibrated GoPro), whereas the floor-plane route uses nothing beyond the already-working parallax term.

## 4.1 Order of operations (call graph)

run_dual.sh  A-5b B-5

└─ python track_dual.py --video-a capture/A-5b.MP4 --video-b capture/B-5.MP4 --outdir output --name A-5b_B-5 ...

main()

├─ process_clip(video_a, ..., write_outputs=False)  -> res_a   (whole of section 2, camera A)

├─ process_clip(video_b, ..., write_outputs=False)  -> res_b   (same, camera B)

├─ world.fit_world(res_a.floor, res_b.floor, 1, 5)  -> wf      shared sheets -> A->B rigid, world frame

├─ crossguide.guide(res_a <- res_b)   \  each camera searches its unfixed frames

├─ crossguide.guide(res_b <- res_a)   /  where the other camera saw the car

├─ relocate_vehicle(res_a) / (res_b)      redo the vehicle stage with the new fixes

├─ write_clip_outputs(res_a), (res_b)      output/<name>/<A>/ and /<B>/ as run_tracking.sh would

├─ fuse.solve_tag_height(...)              tag height that best reconciles the cameras (reported)

├─ fuse.fuse(res_a, res_b, wf.a_to_world, wf.b_to_world, ...)  -> f : Fused

│    ├─ _cam_on_grid ×2  (re-parallax, to world, resample onto the 30 Hz grid)

│    ├─ _merge           (weighted mean where both, single where one; disagreement)

│    └─ kalman_smooth    (default)  or  vehicle.smooth_xy (--fusion mean)

├─ fuse.print_fusion_summary(f)

├─ dual_outputs.write_fused_csv     <name>_fused.csv

├─ dual_outputs.write_topdown       <name>_topdown.png

└─ dual_outputs.write_dual_overlay  <name>_overlay.mp4   (reads both videos again)

## 4.2 run_dual.sh — the two-camera runner

- Takes exactly two clip names. Recognises --fallback, --solve-tag-height, --start-a N, --start-b N itself; everything after -- goes to Python. NAME=Dual1 ./run_dual.sh A-1 B-1 overrides the output folder name, which defaults to <A>_<B>.

- Runs track_dual.py --video-a … --video-b … --outdir ../output --name <NAME> --stride 2 --smooth 15 --tag-heights 20:175 --no-fallback [--solve-tag-height] <extra> and logs to output/<NAME>/<NAME>.log.

- A run takes about 3 minutes per pair; six pairs in parallel took 16 m 44 s (each job already multi-threads, so parallelism gains only ~25 %). About 1 GB RAM each, no memory cap needed.

## 4.3 world.py — one frame for two cameras

Each camera's floor fit is already metric (scale from its 150 mm sheets), but its origin is its own anchor sheet's corner and its axes are that sheet's edges. Two cameras therefore differ by a 2-D rigid motion (rotation + translation), found here from the corners of every GCP that both cameras accepted. The world frame is then defined by the sheet layout — origin at the centre of GCP 1, +x toward the centre of GCP 5 — so that the mapping side can adopt the same definition on the same sheets, which metric_scale.py now does (5.5).

gcp_corners_mm(floor) -> dict[id -> 4×2 mm]

Every accepted sheet's undistorted median corners pushed through floor.H: the sheet layout in this camera's anchor frame.

kabsch_2d(src, dst, scale=False) -> (s, R, t)

Closed-form least-squares fit of dst ≈ s·R·src + t (Kabsch / Umeyama): centre both point sets, SVD of the cross-covariance PᵀQ, R = V·diag(1, det)·Uᵀ with the determinant correction so a reflection is never returned, optional scale s = trace(Σ·D) / Σ|P|², and t from the centroids. With scale=False (the default) it is a rigid fit and s = 1.

When this was written it was expected that contribution 1 (metric scaling of the SfM cloud) would need the same fit in 3-D. It did not: without a surveyed position for each sheet there are no world coordinates to align to, so metric_scale.py takes the scale from the sheets' known side lengths and the frame from the floor plane and two sheet centres (5.5). A 3-D Umeyama fit becomes useful only once a GCP survey exists.

class Rigid2D

A rotation R (2×2) and translation t (2,) that can be called on any point array (p @ Rᵀ + t), inverted with .inv(), and composed with .then(other) (self first, then other). Everything in the two-camera code that moves points between frames is a Rigid2D.

class WorldFit  (dataclass)

shared: GCP ids seen by both cameras. a_to_b: A's anchor frame → B's anchor frame. resid_mm: per shared corner residual after the rigid fit (the GCP join number). scale_ab: the similarity-fit scale B/A, reported only (1.0 = the cameras agree on metric scale; A-5b/B-5 gives 0.94, i.e. camera A measures 6 % large — the camera A lens problem). b_to_world, a_to_world: the two Rigid2Ds to the world frame. layout: id → 4×2 corners in world mm, the mean of both cameras' views for shared sheets.

frame_from_layout(layout, origin_id, x_id) -> Rigid2D

Builds the map from a layout's frame to the world frame: origin at the centre of origin_id, x-axis the unit vector toward the centre of x_id, y-axis its perpendicular with handedness kept. R has the world axes as rows; t = −R·o.

fit_world(floor_a, floor_b, origin_id, x_id) -> WorldFit

Takes the two sheet layouts, intersects their ids (needs at least 2 shared sheets, i.e. 8 corners), stacks the shared corners and calls kabsch_2d twice (rigid for the transform, similarity for the scale diagnostic). Builds layout_b: for shared sheets the mean of A's corners moved into B and B's own; B-only sheets as B saw them; A-only sheets moved into B. Checks the two frame-defining sheets exist, builds b_to_world, moves the layout into world, and derives a_to_world = a_to_b.then(b_to_world).

Prints the WORLD FRAME block: shared ids and corner count, the A→B rotation angle and residual RMS / max, the scale ratio, and a table of every sheet's world centre with which camera(s) saw it and its per-sheet A-B RMS. A residual of tens of mm means the two floor fits agree; hundreds means one of them is wrong — stop and look at the floor fits.

## 4.4 fuse.py — one track from two

Each camera's run gives a metric, parallax-corrected track in its own anchor frame plus a global time per analysed frame. This module moves both into the world frame, resamples them onto one 30 Hz grid, merges them, and smooths the result. Smoothing happens after the merge so a dropout in one camera never drags the other's data through a filter window.

class CamTrack / class Fused  (dataclasses)

CamTrack: one camera's fixes on the common grid — xy (n, 2) world mm with NaN where not located, tagpx tag side in pixels (NaN where not an ArUco fix), frame the nearest source-video frame number (−1 outside the clip; used by the overlay to fetch the right frame).

Fused: the grid times t, the two CamTracks a and b, xy_raw (merged, unsmoothed), xy (merged, smoothed), src per sample ('A', 'B', 'AB', 'K' for Kalman-bridged, ''), have, disagree (|A − B| in mm where both, NaN elsewhere) and tag_h (the tag height used).

_tag_px(res, veh_ids) -> ndarray

Tag side length in pixels per analysed frame, from the corners stored in det.corners_by_frame for whichever id veh_src says was used. This is the weight of a fix: a bigger tag means the car is nearer that camera and the fix is better conditioned.

_anchor_mm(res, h_old, h_new) -> ndarray

Returns the raw fixes in the camera's anchor frame (mm_raw + origin), with the parallax correction undone for h_old and redone for h_new: raw = nadir + (mm − nadir) × H/(H − h_old) then nadir + (raw − nadir) × (H − h_new)/H. Exact and needs no re-detection, which is what lets solve_tag_height() scan the tag height cheaply.

_on_grid(t_src, vals, have, grid, step) -> ndarray

Linear interpolation of vals (sampled at t_src) onto grid, but only between two consecutive located samples — never across a dropout. Three cases: a grid point exactly on a source sample copies it if located; between two samples it interpolates only if both are located; the last sample is handled explicitly. Everything else is NaN. Works for (n,) and (n, 2) arrays.

_cam_on_grid(res, to_world, veh_ids, h_old, h_new, grid, step, fps) -> CamTrack

Puts one camera on the grid: _anchor_mm → to_world on the located frames → _on_grid for positions and tag sizes → nearest source frame number for each grid time.

_merge(a, b) -> (xy, src, have, disagree)

Where both cameras have a fix, the position is the tag-size-weighted mean; a located frame with no tag size (a blob fill) gets that camera's median weight. Where one has a fix, that one is used. Records the source label and the disagreement |A − B| where both. This is the --fusion mean result before smoothing, and the disagreement series is the same for both fusion methods.

kalman_smooth(a, b, step, max_gap, sigma_mm=25, ref_px=100, accel_mm_s2=4000) -> (xy, filled, have)

The default fusion. A constant-velocity Kalman filter over the state [x, y, vx, vy] with a forward pass and a Rauch-Tung-Striebel backward pass. Process noise is built from a white-acceleration model with σ = 4000 mm/s² (the car can change velocity by ~130 mm/s per 33 ms sample). Each camera's fix at a grid time is a separate measurement of position with noise σ = 25 mm × max(100 / tag_px, 0.5) — a 35 px tag at the far end of a view is treated as ~3× noisier than a 100 px one, so the nearer camera dominates automatically. Both cameras' measurements are applied in the same update step when both exist.

Frames with no measurement are predicted, and the predictions are kept only inside gaps of at most max_gap samples (15 = half a second): the prediction is honest over half a second, not over a lap. Those samples are flagged 'K' in the CSV. The backward pass makes every estimate two-sided so nothing lags. Returns the smoothed positions, the mask of predicted-only samples, and the combined coverage mask.

On the six pairs this raised coverage to 94–98 % (from 69–92 % 'either') with less jitter than the Savitzky-Golay alternative.

fuse(res_a, res_b, a_to_world, b_to_world, veh_ids, tag_h_used, tag_h, stride, smooth, method='mean', max_gap=15) -> Fused

Refuses without a timecode on both clips or with different frame rates. Builds the grid: camera A's sample times, extended at the same step (stride / fps = 33.4 ms) to cover B's span as well. Calls _cam_on_grid for both (re-correcting the parallax from tag_h_used to tag_h), _merge, then either kalman_smooth (marking bridged samples 'K') or smooth_xy on the merged raw track. track_dual.py passes method=args.fusion (default kalman).

solve_tag_height(res_a, res_b, a_to_world, b_to_world, veh_ids, tag_h_used, stride, lo=0, hi=400) -> (h_best, rms_used, rms_best)

The parallax term scales each camera's fix about its own nadir. The two nadirs are at opposite ends of the track, so a wrong tag height pushes the two estimates apart along the baseline and the right one brings them together. This function minimises the RMS A-B disagreement over the tag height with a bounded scalar search (0–400 mm, 0.5 mm tolerance), calling fuse() with smooth = 0 at each trial. Always reported in the log; applied to the fused track only with --solve-tag-height.

It found 167 / 177 / 206 mm on the three A1 pairs before the 175 mm value was looked up, and 168–229 mm across all six — a validation result. It is not a substitute for measuring the tag.

print_fusion_summary(f, stem_a, stem_b)

The TWO-CAMERA FUSION block: grid length and step, located-by-A / B / both / either counts and percentages, Kalman-bridged count and final coverage, A-B disagreement RMS / median / 95th percentile / max over the co-visible samples, the tag height used, and the fused path length. The A-B disagreement is the accuracy figure — there is no other ground truth.

## 4.5 crossguide.py — each camera helps the other

Camera B loses the tag for tens of seconds when the car is at the far end of its view (the tag is ~40×15 px there); camera A sees that end well. Once the two floor frames are joined, A's fix at any instant can be projected into B's image and B's detector can take a windowed, upscaled second look exactly there. Same the other way round.

_world_track(res, to_world) -> (t, xy_world, have)

This camera's located frames in world mm (anchor frame → world) with their global times.

_px_of_world(xy_w, res, to_world, tag_h) -> px

The full inverse chain: world → this camera's anchor frame (to_world.inv()) → undo the parallax correction so the point is where a tag tag_h above the floor images (nadir + (mm − nadir) × H/(H − h)) → inverse homography to undistorted pixels → geometry.distort to raw pixels. That is where the tag's image centre should be.

_expected_side_px(px, res, S=150) -> float

How big a 150 mm square would look at that image position, from the homography: push the point and two points 150 mm along x and y through inv(H) and average the two side lengths. Used to set the upscale factor of the window search.

guide(target, target_to_world, source, source_to_world, veh_ids, tag_h, stride, gate_px=220) -> int

For every analysed frame of target that has no vehicle fix, interpolates source's world track at that global time (only between two located source samples), projects it into target's pixels, skips it if off-image, and records (predicted px, expected side) in targets. Then detector.search_frames() reads the target clip once and searches only those frames. Hits are inserted into target.det.corners_by_frame in place and target.det.recovered is extended; the caller (track_dual.main) then calls relocate_vehicle() to rebuild the track. Returns the number of frames that gained a fix.

Both directions are guided from the other camera's first-pass fixes, so neither depends on the order; a second round would chase the first round's finds. Results are modest — on A-5b/B-5, 32 + 27 frames — because the limit is tag pixels, not search: the projection lands within ~30 px of the car at 5 m (a validation of the geometry in itself) but there are no bits to decode. It will pay off with a bigger tag or higher cameras. --no-cross-guide skips it and saves one extra decode pass per clip.

## 4.6 dual_outputs.py — writing the two-camera results

write_fused_csv(path, f)

One row per grid sample: time_global_s, source (A/B/AB/K/empty), x_mm, y_mm (fused, world frame), xA_mm, yA_mm, xB_mm, yB_mm (each camera's own world-frame position on the grid), disagree_mm, tagpx_A, tagpx_B, frame_A, frame_B (source frame numbers). Empty cells where a value does not exist.

write_topdown(path, f, wf, title)

A deliberately plain matplotlib plot in world mm: every sheet from wf.layout as a grey square with its id, camera A fixes as small orange dots, camera B fixes as blue dots, and the fused smoothed track as a line broken at dropouts (NaN inserted where have is false, so no chord is drawn across a gap). Equal aspect, grid, legend with counts. Kept minimal on purpose — the plot restyle is Antwan's.

_world_to_px(xy_w, res, to_world) -> px

World mm → this camera's raw pixels: to_world.inv() → inv(H) → distort. Note it does not undo the parallax (unlike crossguide._px_of_world): the fused track is a floor track and is drawn where that floor point is in the image, i.e. under the car, not on the tag.

class _Seq

A sequential video reader that can be asked for any non-decreasing frame number: it reads forward until it reaches the requested frame and returns it, or None past the end or for a negative request. Avoids CAP_PROP_POS_FRAMES seeking, which is unreliable on HEVC.

write_dual_overlay(path, f, res_a, res_b, wf, panel_w=1920)

Both views side by side (2 × 1920 wide, height from A's aspect), one output frame per grid sample at the grid rate. For each sample and each camera: fetch the source frame f.a.frame[k] (a black panel labelled 'not recording' if that camera was not rolling), draw the static GCP outlines, draw the fused track reprojected into this camera's pixels as yellow polylines one per unbroken run (so a dropout is a gap, not a chord), a yellow ring at the current fused position, a filled red dot at this camera's own fix for its nearest analysed frame, and a label. A HUD across the top shows the global time, the source label and the A-B disagreement in mm.

This is the file to open first after a run. Antwan's verdict on the first set: 'looks amazing'.

## 4.7 track_dual.py — the two-camera driver

parse_args()

--video-a, --video-b, --outdir, --name, --world-origin (1), --world-x (5), --start-a, --start-b, --lens-a, --lens-b, --fusion kalman|mean (kalman), --max-gap (15), --no-cross-guide, --solve-tag-height, plus every flag from add_tracking_args().

main()

Makes two shallow copies of the args with the per-camera start_s and lens filled in, runs process_clip on each without writing outputs, prints both clip start times and their difference (a sanity check on the timecode: A-5b starts 36.55 s after B-5), fits the world frame, runs cross-guidance both ways and relocates whichever camera gained fixes, then writes both single-camera output sets into output/<name>/<A>/ and /<B>/. Solves and reports the tag height, chooses h_fuse (the solved value only with --solve-tag-height), fuses, prints the summary and writes the CSV, top-down and overlay.

## 4.8 calibrate_lens.py — pooled lens self-calibration

Each clip's floor fit estimates distortion from the 4–6 sheets it can see, all sitting in one region of the frame — under-determined, and it showed: camera A's k1 pinned at the +0.5 bound with a 30 mm sheet residual at position A2. The lens does not change between clips, so this tool pools them: every fixed clip contributes one pose (its median sheet corners) and a handheld mapping walk contributes a pose per sampled frame with sheets scattered across the whole image. One (k1, k2), optionally with the principal point, is fitted to make every sheet in every pose square at once.

sample_poses(video, gcp_ids, stride, min_sheets) -> (poses, (W, H), kind)

Decodes every stride-th frame (30 by default), keeps only GCP ids. Decides fixed vs handheld by whether the sheets move (the same 90th-percentile-spread-under-5-px test as fit_floor): fixed → one pose from the median corners if it has at least min_sheets; handheld → each sampled frame with at least min_sheets is a pose.

pose_error(pose, c, s, k1, k2, S) -> float

square_error of one pose under a lens model, taking the best anchor for that pose — so the lens fit is not hostage to one bad anchor choice.

fit(poses, W, Hh, S, fit_centre) -> (k1, k2, c, s, rms_after, rms_before)

Nelder-Mead over (k1, k2[, cx, cy]) from three starts, minimising the sheet-count-weighted RMS of pose_error over all poses, with the same plausibility guards as fit_floor. --fit-centre needs varied poses (a walk) to be determined.

main()

Runs sample_poses on every video given (refusing mixed resolutions), fits, prints the coefficients, residual before/after, the division factor at six radii, and a warning if k1 sits at the bound; writes the JSON that --lens / --lens-a / --lens-b consume.

What is in calib/ now, from 21 September:

| File | Poses | k1 / k2 | Residual before → after | Reading |
| --- | --- | --- | --- | --- |
| camA.json | 6 fixed clips (A-1…A-5b), 4–5 sheets each | +0.273 / +0.051 | 11.9 → 7.2 mm | Positive k1 — opposite sign to B. Camera A's images do not fit pinhole + radial well. |
| camB.json | 7 fixed + 2 handheld walks = 172 poses | +0.065 / −0.029 | 22.6 → 22.4 mm | Barely improves: the handheld walk frames carry rolling-shutter error, which is not radial. |
| camB_fixed.json | fixed clips only | — | — | The variant without the walks. |

Conclusion recorded in the plan: the pooled fit is built but not a default win on this footage. The per-clip fit remains the default; --lens is available for experiments. The real fix is a ChArUco calibration of both bodies in the identical lens mode, still to be shot.

## 4.9 Reading a two-camera log — A-5b_B-5 as a worked example

anchor candidates : 0: 2400.5 mm, 1: 13.1 mm, 5: 17.3 mm  -> 1        camera A: the largest sheet (0) would have been a disaster

FLOOR PLANE FIT   anchor ID 1, 150.0 mm assumed

distortion fit    : k1 =+0.49995  k2 =-0.00514   (RMS 25.10 -> 13.09 mm)   k1 at the +0.5 bound: camera A's lens problem

focal length      : 2026 px   (two constraints: 2024 / 2028 px)

camera height     : 2080 mm recovered from the homography                   do NOT read this as A's real height; the fit is poor

anchor candidates : 2: 40.7 mm, 3: 6.0 mm, 4: 9.0 mm, 5: 4.0 mm  -> 5      camera B

distortion fit    : k1 =-0.13357  k2 =+0.02184   (RMS 8.20 -> 4.01 mm)      healthy barrel correction, 4 mm residual

focal length      : 1814 px   (two constraints: 1798 / 1831 px)

camera height     : 3221 mm recovered from the homography                   B2 vs the 3271 mm reference: -1.5%

clip starts       : A 15:33:33.967   B 15:32:57.417   (B - A = -36.550 s)

WORLD FRAME   origin GCP 1 centre, +x toward GCP 5

shared GCPs       : [0, 1, 2, 4, 5]  (20 corners)

A -> B rigid fit  : rotation +7.8 deg, residual rms 109.9 mm, max 173.0 mm   the "GCP join" — dominated by camera A's fit

scale B/A         : 0.9398  (-6.02% ...)                                     camera A measures 6% large

cross-camera guide: A-5b <- B-5: 912 frames searched, tag found in 32

cross-camera guide: B-5 <- A-5b: 88 frames searched, tag found in 27

vehicle fixes     : A 1149 -> 1181   B 3047 -> 3074

tag height solve  : 175 mm gives A-B rms 64.1 mm; best agreement at 168 mm (rms 61.7 mm)

located by A      :  1140 ( 33.7%)      located by B :  2946 ( 87.2%)

located by both   :  1028 ( 30.4%)      located by either : 3058 ( 90.5%)

bridged by Kalman :   237 (  7.0%)  -> track covers 3295 (97.5%)

A-B disagreement  : rms 64.1 mm, median 63.4 mm, 95% 93.1 mm, max 212.6 mm  (over 1028 co-visible samples)

fused path length : 98342 mm (98.34 m)

Results across all six pairs, 21 September, tag 175 mm, all improvements on:

| Pair | Positions | Fix rate A / B | GCP join RMS | Co-visible | Either → Kalman | A-B RMS / median | Solved tag h |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A-1_B-1 | A1/B1 | 58 % / 16 % | 40 mm | 1.9 % | 69 → 72 % | 215 / 210 mm | 229 mm |
| A-2_B-2 | A1/B1 | 71 % / 59 % | 13 mm | 23 % | 87 → 98 % | 162 / 145 mm | 200 mm |
| A-3_B-3 | A1/B1 | 51 % / 64 % | 48 mm | 13 % | 92 → 98 % | 147 / 92 mm | 174 mm |
| A-4_B-4 | A2/B1 | 38 % / 60 % | 54 mm | 5 % | 83 → 94 % | 196 / 139 mm | 217 mm |
| A-5_B-5 | A2/B2 | 50 % / 90 % | 91 mm | 6.5 % | 87 → 97 % | 66 / 61 mm | 175 mm |
| A-5b_B-5 | A2/B2 | 54 % / 91 % | 110 mm | 30 % | 91 → 98 % | 64 / 63 mm | 168 mm |

The B2 pairs (overhead, good view) agree to 64–66 mm RMS; the B1 pairs (low, far) to 147–215 mm. Camera A is the weak camera at every position.

# 5. Pipeline B — track mapping

A handheld GoPro is walked slowly around the track. Frames are extracted at 4 fps, COLMAP finds SIFT features in every frame, matches consecutive frames, and its incremental mapper solves for every camera pose and a sparse 3-D point cloud (structure from motion). OpenMVS then computes a depth map per view and fuses them into a dense cloud. Optionally OpenSplat trains a Gaussian splat for viewing. All of that is in SfM units: the reconstruction is correct only up to an unknown scale, rotation and translation. The boundary stage (run_boundary.sh, 28 September) closes that gap and delivers the two contributions: metric_scale.py finds the 150 mm floor sheets in the frames and turns the model into millimetres in the tracker's world frame, boundary.py extracts the track walls from the metric dense cloud, and boundary_overlay.py draws the result back into the video as a check.

## 5.1 Order of operations

run_mapping.sh  GX011488 GX011489            (NAME defaults to GX011488_GX011489)

source lib.sh

resolve_clip ×2 ; set_name ; write_sources

ensure_frames        extract_frames.py per clip @ 4 fps -> capture/<NAME>_work/frames/w1_*.png, w2_*.png (hard links)

── sparse ──          (skipped if output/<NAME>/sparse/points3D.bin exists and FORCE!=1)

systemd-run -p MemoryMax=20G  colmap_pipeline.py --max-image-size 3200 --num-threads 8 --peak-threshold 0.003

--camera-model OPENCV --single-camera [--exhaustive]

colmap feature_extractor -> sequential_matcher | exhaustive_matcher -> mapper

largest_model        pick the sparse/N with the most registered images  -> output/<NAME>/sparse/

model_stats ; colmap model_converter -> <NAME>_sparse.ply ; plot_topdown.py -> <NAME>_topdown.png

[SPARSE_ONLY=1 -> cleanup_work, exit]

── dense ──

run_dense_stage      dense_reconstruction.py --resolution-level 2 (retry 3) --number-views 5 --max-threads 16 --memory-limit 24G

colmap image_undistorter -> colmap model_converter TXT -> docker InterfaceCOLMAP -> docker DensifyPointCloud

-> output/<NAME>/<NAME>_dense.ply

cleanup_work           rm -rf capture/<NAME>_work

run_dense.sh <NAME>      read_sources ; ensure_frames ; run_dense_stage ; cleanup_work

run_splat.sh <NAME> [d]  read_sources ; ensure_frames ; memory check ; docker opensplat-rocm7 ; cleanup_work

run_boundary.sh <NAME>   read_sources ; ensure_frames                      (needs sparse/ and <NAME>_dense.ply)

metric_scale.py      detect GCPs in every registered frame -> triangulate 24 corners -> scale,

hold-out, floor plane, world frame                  -> <NAME>_metric.json/.log

boundary.py          dense cloud -> mm/world -> crop -> floor -> wall band -> raster -> DBSCAN ->

centrelines -> midline + width -> track cross-check -> csv/dxf/json/png/ply

boundary_overlay.py  boundary lines -> SfM units -> every 2nd frame via its COLMAP pose -> mp4 + jpg

cleanup_work

## 5.2 lib.sh — the shared helpers

Sourced, not run, by the four runners. It defines the layout (capture/<video>, transient capture/<NAME>_work/, deliverables in output/<NAME>/) and these functions:

say <text>

Timestamped echo that also appends to $LOG once set_name has set it.

resolve_clip <name-or-file>

Name → absolute path of capture/<name>.MP4 (any case of extension). On failure prints the names that do exist.

set_name <clip> [<clip> …]

Sets NAME (the stems joined with _, unless NAME= was given), OUT=output/<NAME>, WORK=capture/<NAME>_work, FRAMES=$WORK/frames, LOG, and creates OUT.

write_sources / read_sources

Write the clip paths that built a model to output/<NAME>/sources.txt; read them back into CLIPS. This is how run_dense.sh and run_splat.sh can re-extract frames that a completed run deleted.

ensure_frames <clip> …

If $FRAMES already has files, do nothing. Otherwise extract each clip with extract_frames.py into $WORK/clips/<stem>/ and hard-link every frame into $FRAMES/w<i>_frame_NNNNNN.png — hard links, not symlinks, because symlinks do not resolve inside the OpenMVS Docker container; the w<i>_ prefix keeps the source clip visible in every filename. Removes $WORK/clips.

largest_model <sparse-parent>

Runs colmap model_analyzer on every sparse/N/ and returns the one with the most registered images. This exists because COLMAP writes one folder per connected component and does not sort by size — sparse/0 has been a 2-image fragment while sparse/1 held 213 images. OpenSplat always reads sparse/0, so run_splat.sh copies the largest one there.

model_stats <model>

Greps registered images, points, mean track length and mean reprojection error out of model_analyzer.

dense_attempt <model> <level>

Deletes stale .dmap files (they are resolution-specific but reused by name), then runs dense_reconstruction.py at the given resolution level with a 24 GB Docker cap and 16 threads.

run_dense_stage <model>

Tries level 2; if that fails (fusion memory is the usual cause) retries at level 3; copies scene_dense.ply to output/<NAME>/<NAME>_dense.ply and reports the point count.

cleanup_work

rm -rf $WORK — capture/ holds only the videos again. On a failure the runners deliberately skip this so the work folder can be inspected.

## 5.3 The four runners

run_mapping.sh <name> [<name> …]

The full pipeline as in 5.1. Environment toggles: NAME= (model name), SPARSE_ONLY=1 (stop after sparse and the top-down plot — recommended first on any new scene, so the component count is read before an hour of dense is spent), EXHAUSTIVE=1 (match every image pair rather than sequential neighbours, for a model that fragmented), FORCE=1 (redo the sparse even if one exists), FPS= (extraction rate, default 4).

The COLMAP stage runs under systemd-run --scope -p MemoryMax=20G if systemd-run exists. If the mapper produced more than one component the log says model is FRAGMENTED — that means the walk lost tracking somewhere.

Settings and why each exists: fps 4 gives inter-frame motion of 1–5 % of image width, well inside the <10 % rule for sequential matching. --num-threads 8 because each SIFT worker holds a decoded 4K image plus its pyramid (~1.2 GB) and 24 threads reached 28 GB and got the session OOM-killed. --max-image-size 3200 --peak-threshold 0.003 are what made SIFT work on a uniform tiled floor (features per image 1.7 k → 4–10 k, registration → 100 %). --camera-model OPENCV --single-camera because it is one GoPro with shared intrinsics.

run_dense.sh <NAME> [<NAME> …]

Dense only, for models that already have output/<NAME>/sparse/. Rebuilds the frames from sources.txt (~15 s per 4K clip), runs run_dense_stage, deletes the frames again. Loops over several names.

run_splat.sh <NAME> [downscale=3] [iterations=30000]

Gaussian splat with the AMD/ROCm opensplat-rocm7 Docker image. Rebuilds frames, copies the sparse model into a project folder with sparse/0 (OpenSplat's expectation), and refuses to start a run that cannot fit: the resident image cost is 2.06 × images × (w/d) × (h/d) × 3 × 4 bytes (the 2.06 is measured — OpenSplat holds about two float32 copies per image), and at its default -d 1 558 4K frames would need 106 GB. The container gets --memory = 1.6× that plus 4 GB, and --device /dev/kfd --device /dev/dri plus the render and video groups, without which torch silently trains on the CPU. Output in output/<NAME>/splat_d<D>/.

run_boundary.sh <NAME> [<NAME> …]

Metric scale, boundary and overlay for a model that run_mapping.sh and run_dense.sh have already built. It refuses a name without output/<NAME>/sparse/points3D.bin or without <NAME>_dense.ply. It does not call set_name (there are no clips on its command line); it sets OUT, WORK, FRAMES and LOG itself and uses read_sources + ensure_frames to rebuild the frames from sources.txt. The frames are needed because metric_scale.py looks for the sheets in the very images COLMAP registered, matched by file name (w1_frame_000123.png), and boundary_overlay.py draws on them.

Then three stages, each logged to its own file and appended to <NAME>.log: metric_scale.py (its detect N/577 progress lines are filtered out of the log with grep -v), boundary.py with the track glob, and boundary_overlay.py, which needs the wall height that boundary.py measured; the runner reads wall_top_mm back out of <NAME>_boundary.json with a one-line Python call. Finally cleanup_work. Environment: WORLD_ORIGIN (1) and WORLD_X (5) choose the frame-defining sheets and must match what run_dual.sh used; TRACKS is the glob of fused trajectory CSVs for the corridor check, by default ../pipeline_A_tracking/output/*/*_fused.csv, i.e. every two-camera run.

Cost: 56 s for TrackMap_S2 on 28 September with the frames already present (about 13 s scale, 9 s boundary, 34 s overlay); rebuilding the frames adds roughly 15 s per 4K clip. No systemd-run memory cap is used for this stage.

## 5.4 The Python stages

extract_frames.py  — extract_video(video_path, out_root, fps) -> out_dir

ffmpeg -i <video> -vf fps=<fps> <out>/<stem>/frame_%06d.png. Nothing else; the CLI wraps it and prints the count.

colmap_pipeline.py  — main()

Three colmap subprocess calls in sequence. feature_extractor with --SiftExtraction.use_gpu 0/1 (--gpu uses SiftGPU over OpenGL, which works on the AMD card but only from a real display; not used), max_image_size, num_threads, optional peak_threshold, ImageReader.camera_model, ImageReader.single_camera. Then sequential_matcher or exhaustive_matcher (--exhaustive). Then mapper with the same thread count, writing sparse/0, 1, …. Defaults in the script (--max-image-size 1600) are the old 15 GB-machine values; the runner overrides them.

dense_reconstruction.py  — run(), run_openmvs(), main()

run(cmd, memory_limit) runs a native command inside systemd-run --user --scope -p MemoryMax=<limit> -p MemorySwapMax=0 so a runaway is killed on its own rather than by the system OOM killer against the whole desktop. run_openmvs(workspace, tool, args, memory_limit) runs an OpenMVS binary from the openmvs/openmvs-ubuntu image with the workspace mounted at /workspace, as the current user (so outputs are not root-owned), under --memory/--memory-swap.

main() chains: colmap image_undistorter (OpenMVS only supports the PINHOLE model, so images and model are undistorted first) → colmap model_converter --output_type TXT (InterfaceCOLMAP reads only the TXT export) → InterfaceCOLMAP (TXT → scene.mvs) → DensifyPointCloud --resolution-level N --number-views 5 --max-threads 16 (→ scene_dense.ply). The memory reasoning: OpenMVS fusion holds every depth map resident at once, so memory ≈ views × depth-map pixels; half-res × ~450 views needs ~18 GB, quarter-res (level 2) ~14 GB and completes.

plot_topdown.py  — fit_plane(), plane_basis(), main()

fit_plane(P, iters=800, thresh_frac=0.01): RANSAC over 3-point samples for the dominant plane; the inlier threshold is 1 % of the cloud's largest extent so it does not need retuning per scene (and is kept tight on purpose — a loose threshold calls everything an inlier and the '99 % inliers' means nothing). Returns normal, offset, inlier mask. plane_basis(n) gives two orthonormal in-plane axes. main() loads the model with pycolmap.Reconstruction, projects points and camera centres into the plane, trims 0.5 % off each end of the extent so stray points do not set the axes, and renders a scatter coloured by height above the floor with the camera path, start and end. The title carries registered / total, point count, mean reprojection error and floor-inlier fraction — a healthy walk has 100 % registered in one component, ≥ 95 % floor inliers, and a camera path that closes on itself.

The file's own docstring notes the orientation of the plot is arbitrary (the in-plane axes come from the normal, not from the track). That is still true of <NAME>_topdown.png, which remains the quick health check of a sparse model. The metric, world-frame top-down is now <NAME>_boundary_topdown.png from boundary.py. The boundary stage does not reuse this RANSAC plane: it starts from the plane through the triangulated sheet corners (5.5) and refits the dense floor by least squares (5.6).

## 5.5 metric_scale.py — millimetres and the world frame (contribution 1)

An SfM model is correct only up to a similarity: an unknown scale, a 3-D rotation and a translation, seven numbers in all. The six 150 mm sheets lying on the floor fix all seven without a survey. Their side lengths give the scale, the plane they lie in gives "up", and two of them give the origin and the direction of x. Because those are the same two sheets that world.frame_from_layout() uses (origin at the centre of GCP 1, +x toward GCP 5), the map lands in the very frame the two-camera trajectories are already in.

The module reuses pipeline A's tuned detector: it puts pipeline_A_tracking/src on sys.path and imports build_detector, so there is one detector definition for both pipelines. GCP_IDS is 0–11; the vehicle tag is never looked for here.

_detect(path) -> (image name, {id: 4×2 corners px})

Runs in a worker process. The detector is built lazily once per process in a module global, because an OpenCV detector object cannot be pickled and sent to a worker. Reads one extracted PNG in grey, detects, and keeps only GCP ids. Returns the file name (the key COLMAP knows the image by) and the raw-pixel corners in ArUco order.

_dlt(Ps, xs) -> X

Linear triangulation (direct linear transform) of one point from two or more views. Each view contributes two rows, x·P₃ − P₁ and y·P₃ − P₂, where P is the 3×4 cam_from_world matrix and (x, y) the normalised image coordinates; the point is the right singular vector of the stacked matrix with the smallest singular value, dehomogenised. Normalised coordinates come from camera.cam_from_img(), which removes both the intrinsics and the OPENCV-model lens distortion that COLMAP estimated, so the linear model holds.

triangulate(obs, thresh_px, min_views, rng) -> (X, inlier idx, rms px) | None

obs is every sighting of one sheet corner: (P, normalised xy, pixel xy, camera) per registered frame that saw it, 181–346 views per corner on TrackMap_S2. A plain DLT over all of them would let one bad sighting (a partly hidden sheet, motion blur, a corner refined onto the wrong edge) pull the point, so it is done in two stages.

RANSAC over view pairs. Every pair of views (or 300 random pairs when there are more, drawn with a fixed seed so a rerun gives the same answer) is triangulated and scored by how many views reproject within thresh_px (2 px) of their detection. Reprojection uses camera.img_from_cam(), i.e. through the lens distortion, so the residual is in real image pixels; a point behind a camera scores 10⁶. The pair with the most inliers wins.

Refine. DLT again on all inliers, then a Levenberg–Marquardt least-squares fit of X that minimises the pixel residual (DLT minimises an algebraic error, which is not the quantity that matters). Inliers are recounted, at least min_views (3) are required, and the RMS pixel residual is returned. The single-camera model (one GoPro, one set of intrinsics) is assumed: the camera of the first sighting is used for all.

sheet_lengths(c) -> 6 lengths

The four sides and two diagonals of a 4×3 array of sheet corners, in whatever unit the corners are in. Six lengths rather than four for the same reason geometry.square_error uses diagonals: they constrain the shape as well as the size.

main()

Flags: --sparse, --images, --out (required); --gcp-mm 150; --origin 1 and --x-to 5 (the frame-defining sheets); --thresh-px 2.0; --min-views 3; --workers 8 (each worker holds one decoded 4K frame; 8 is about 1 GB). It loads the model with pycolmap.Reconstruction and stops if any registered image is missing from --images.

Step 1 — detect. A multiprocessing.Pool maps _detect over every registered frame in name order (chunks of 4). The pool is closed and joined by hand rather than used as a context manager, because the context manager's terminate() sends SIGTERM to the workers and pycolmap's crash handler then prints a stack trace. Prints how many frames saw each sheet (TrackMap_S2: 0:181, 1:292, 2:269, 3:228, 4:346, 5:259).

Step 2 — triangulate. For every (sheet id, corner index) the sightings are collected with that frame's cam_from_world matrix (the code accepts it as a method or a property, which differs between pycolmap versions) and triangulated. A sheet with all four corners is "complete"; a table of every corner with its views, inliers and RMS is printed, with <- FAILED where triangulation gave up. At least two complete sheets are required.

Step 3 — scale. Each complete sheet gives one scale: the median over its six lengths of (nominal length / SfM length), nominal being 150 mm for a side and 150√2 = 212.1 mm for a diagonal. The model scale is the median over sheets. Medians twice, so neither one bad length nor one bad sheet moves the answer. The table shows each sheet's scale, its deviation from the median, and its six lengths in mm at the median scale; the spread (max − min) / median is the first consistency number.

Hold-out. With at least three sheets, each sheet in turn is left out: the scale is recomputed as the median of the other sheets' scales, and the left-out sheet's six lengths are measured with it. That sheet played no part in the scale it is measured with, so its error is an honest estimate of the error on a new 150 mm object anywhere on the floor. This is the accuracy figure for contribution 1, reported as RMS and maximum in mm and as a percentage of 150 mm.

Step 4 — frame. All corners are scaled to mm and a plane is fitted through them by SVD (the normal is the singular vector with the smallest singular value). The normal is flipped if needed so that the median camera centre is above the floor. The plane residual (RMS and maximum) is printed; it contains sheet flatness and triangulation error together. The origin is the centre of --origin projected into the plane, x is the in-plane unit vector toward the centre of --x-to, and y is chosen to match the tracker, which is the subtle part.

The tracker's floor frame puts sheet corner 0→1 on +x and corner 0→3 on +y (2.2). The code averages the cross product (c₁ − c₀) × (c₃ − c₀) over all sheets. If it points with the up normal the tracker frame is ordinary; if against, the tracker frame seen from above is a mirror image, and y is taken as (−n) × x so the map matches it. R has rows (x, y, n), the transform is world = R·(s·X_sfm) + t with t = −R·o, and det R = −1 in the mirrored case (R is then a reflection, still orthogonal, so its inverse is still its transpose). On TrackMap_S2 the frame is mirrored, as expected from 2.2.

Two plausibility prints follow: the hand-held camera's height above the floor along the walk (median 1148 mm, 5–95 % range 696–1296 mm, a plausible height for a hand-held camera), and every sheet centre in world mm with its height, which should be within a few mm of zero.

Written to <NAME>_metric.json: the scale, R and t, the transform as a sentence, tracker_frame_mirrored, per-sheet scales, the spread, the hold-out results, the plane RMS and maximum, every sheet's corners and centre in world mm, the median camera height, the per-corner triangulation table and the sighting counts. Everything downstream reads only R, t and the scale (plus the sheet corners for drawing).

## 5.6 boundary.py — the track walls (contribution 2)

The input is the dense cloud, put into millimetres and the world frame with the transform from 5.5. That is what lets every threshold below be a physical distance set once (30 mm, 10 mm cells, 50 mm) rather than a number tuned per cloud in SfM units. On the session-2 track the boundary is a raised wall of grey bolted panels about 320 mm high with a central island, not tape on the floor; the method extracts the centreline of each wall seen from above, then the corridor between them.

load_metric(dense, sparse, metric) -> (metric dict, P, cams)

Reads the JSON, loads the dense PLY with Open3D and applies world = R·(s·X) + t to every point (16.2 M on TrackMap_S2), and does the same to every registered camera centre from the sparse model (sorted by name). The cameras are only used to crop.

fit_floor(P, band_mm) -> (plane coefficients, rms)

Least-squares plane z = a·x + b·y + c through the points within band_mm (30 mm) of z = 0, then refitted on the points within 30 mm of that plane, and the final RMS is taken over the points within 15 mm of it. Least squares in a band rather than RANSAC because the starting point is already the sheet plane from 5.5, so the floor is the dominant structure near z = 0 and a band-limited fit is deterministic. Its offset and tilt are also a check on 5.5: two independent estimates of the same floor, one from 24 triangulated corners, one from millions of dense points.

class Grid(xy, cell, pad)

The raster geometry used throughout: origin and size of a grid of cell-mm squares covering the points plus pad. idx() turns mm into (row, col), xy() turns (row, col) into cell-centre mm, count() is a 2-D histogram of points per cell (np.add.at).

equidistant_contour(outer_mask, inner_mask, grid) -> N×2 mm

The geometric core, used twice. Given a filled region and a region inside it, it returns the closed line equidistant from the outer edge and the inner region: two distance transforms (distance of each cell to the outer edge; distance to the inner region), keep the cells nearer the inner region than the outer edge plus the inner region itself, and take that region's outer contour. Applied to a wall band (filled band, enclosed hole) it gives the band's centreline; applied to the area between the two walls it gives the track midline. It works on any shape of track, with no assumption about straights or curves.

resample(poly, step) · normals(poly) · seg_dist(pts, poly) · polygon_length(p) · polygon_area(p)

Helpers for closed polylines: equal-arc-length resampling; unit normals from a central-difference tangent; the unsigned distance from points to a closed polyline (densified to 2 mm and queried with a KD-tree, so accurate to about a millimetre); perimeter; and the shoelace area.

snap_to_wall(poly, pts, tree, half_width, reach) -> poly

The equidistant line is a raster estimate (10 mm cells) of a band whose edges are ragged with occupancy noise. This moves every vertex along its normal to the median normal offset of the actual wall points in a strip ±half_width (40 mm) along the wall and ±reach (120 mm) across it, if the strip holds at least 5 points. The median makes it robust to stray points and gives sub-cell precision. It is applied twice.

smooth_closed(poly, step, smooth_mm) -> poly

A periodic smoothing spline (scipy.interpolate.splprep, per=True) through the polyline resampled at 10 mm, with the smoothing factor N·smooth² so the curve may deviate from the vertices by about smooth_mm (10 mm) RMS; evaluated back at 10 mm spacing. A wall seen from above is a smooth curve, and this removes the vertex-to-vertex jitter of the snap without rounding off real corners.

write_dxf(path, polys) · write_walls_ply(path, polys, height)

write_dxf writes a minimal ASCII DXF (R12), units mm ($INSUNITS 4), one closed POLYLINE per loop on a layer named after it (outer, inner, midline). It opens in any CAD program, which is what re-laying the same track another day needs. write_walls_ply extrudes each wall centreline from z = 0 to the measured wall top as a strip of triangles, a light 3-D model of the walls for a viewer.

main()

Flags (defaults): --cell 10 mm, --band-lo 30 mm, --top-margin 20 mm, --close-mm 60 mm, --min-count 3 points per cell, --eps 50 mm, --smooth 10 mm, --snap-width 40 mm, --tracks (glob, empty = no cross-check), plus the required --dense, --sparse, --metric, --out-dir, --name. The steps, each with the reason it exists:

- Crop. Keep the points inside the convex hull of the camera path (x, y). The walk circled the track, so the hull contains it, and the room, the corridor and the far floaters fall away: 16.23 M → 14.50 M points.

- Floor. fit_floor, then every height is re-measured from that plane.

- Band. A 10 mm histogram of heights from band-lo to 1 m. The wall top is the upper edge of the highest bin that still has 5 % of the peak count (320 mm). The band kept is 30 mm to top + 20 mm. The lower limit removes the floor and the sheets. The top rim is deliberately kept: on a thin vertical wall the rim lies on the centreline, and on stretches the walk saw only at a glancing angle the rim and the foot are all the cloud has; without the rim the outer wall had a gap on one side.

- Raster. The band is projected to the floor and counted on the 10 mm grid; cells with at least 3 points are wall (33,420 cells). Working on cells rather than 2 M points makes the clustering cheap and already applies a density threshold.

- DBSCAN. scikit-learn DBSCAN on the occupied cell indices, eps 50 mm (5 cells), at least 4 cells. For each cluster: a morphological closing (an ellipse about 60 mm across) bridges small gaps in the wall, the external contour is filled, and the hole is filled minus wall. Clusters are sorted by area; a "loop" encloses more than 0.1 m². The outer wall is the biggest loop, the inner wall is the first loop lying entirely inside the outer one's hole. The script stops if it does not find exactly that structure: it assumes a closed outer wall and one island. Everything else is dropped and recorded (6 clusters, 162 cells on TrackMap_S2).

- Centrelines. For each wall: the band points in its cells, equidistant_contour → resample at 20 mm → snap_to_wall twice → smooth_closed. Reported per wall: length, enclosed area, extent, and the distance of the wall points within 150 mm to the line (RMS, median, 95th percentile, share beyond 150 mm). That spread is the thickness and blur of the wall in the cloud, not the error of the line: the line is a median through hundreds of thousands of points.

- Corridor. On a finer 5 mm grid, the two wall polygons are filled and equidistant_contour gives the midline; width at each midline vertex is the distance to the outer plus the distance to the inner wall, so it is measured wall centre to wall centre (the clear driving width is narrower by one wall thickness). centring_rms (the RMS difference of the two distances) checks that the midline is centred. The drivable area is the outer area minus the inner area, also to the centrelines.

- Cross-check with the tracker. For every fused CSV in --tracks, each (x_mm, y_mm) is tested for being inside the outer wall and outside the island, and its clearance to the nearer wall is measured (negative when outside). Reported per run: samples, % outside, minimum, 5th percentile and median clearance. The trajectories come from different cameras through different geometry, so the car staying inside the corridor is a consistency check between the two contributions. It is not an accuracy measurement of either.

Outputs: <NAME>_boundary.csv (loop, i, x_mm, y_mm for outer, inner, midline), _boundary.dxf, _walls3d.ply, _boundary.json (every number printed), _boundary_topdown.png (log point density in grey, each car track as coloured dots, the two wall centrelines in red, the midline dashed orange, the sheets in black with their ids) and _width.png (width against distance along the midline). The top-down's title includes "fit rms 28/29 mm": that is the wall blur from the centreline step, and it must not be quoted as the boundary's accuracy.

## 5.7 boundary_overlay.py — the boundary drawn back into the video

The check that a person can judge by eye. The boundary was computed from the dense cloud in millimetres; this module takes it back through the inverse transform and every frame's COLMAP pose into the original images. If scale, frame, floor and lines all agree, the lines sit on the wall in every frame of the walk.

main()

Flags: --sparse, --images, --metric, --boundary, --wall-top (mm, from the boundary JSON), --out; --every 2 (every second registered frame), --width 1920, --fps 8.

Each wall centreline is lifted to 3-D twice, on the floor (z = 0) and at the wall top, and the midline once at z = 0; each loop is closed. They are taken to SfM units with X_sfm = ((w − t)·R) / s, the inverse of world = R·(s·X) + t, which is valid because R is orthogonal even when it is a reflection. For each selected frame (in name order): points into the camera with cam_from_world; points closer than 5 % of the median depth or behind the camera are dropped, because they would project to absurd pixels; the rest are projected with img_from_cam, which applies the lens distortion, so the lines curve exactly as the wall does in the image. A segment is drawn only when both ends survive and lie within ±20,000 px. Floor lines are 5 px, top lines 3 px, both red; the midline is 3 px orange. There is no occlusion test: a wall hidden behind another is drawn anyway.

Frames are resized to 1920 wide, captioned, and written as JPEGs to a hidden temporary folder (.overlay_*) inside the output folder, then encoded with ffmpeg (H.264, CRF 22) at 8 fps. The frame one third of the way through is kept as <NAME>_boundary_overlay.jpg, a still for the report; the temporary folder is removed. TrackMap_S2: 289 frames, 36 s of video.

Reading it: the thick line should sit on the foot of the grey wall and the thin line on its rim. On 28 September this was checked on the still; the MP4 is the file to open first after a boundary run, the way the two-camera overlay is for tracking.

## 5.8 Reading a boundary log — TrackMap_S2 as a worked example

The 28 September run, ./run_boundary.sh TrackMap_S2, annotated. The log also contains one pycolmap stack trace after the model: line (a worker receiving SIGTERM as the pool shuts down); the results after it are complete, so it is noise, not a failure.

model: 577 registered images, 66,439 points

detect 577 frames; GCP sightings: 0:181, 1:292, 2:269, 3:228, 4:346, 5:259    all six sheets, 181-346 views each

triangulated 24 corners; complete sheets: [0, 1, 2, 3, 4, 5]                 every corner 0.73-1.24 px rms

SCALE   840.630 mm per SfM unit   (median of 6 sheets)

4   843.168     +0.30%    150.3  148.4  150.1  149.4  211.2  211.7          the outlier sheet: +0.3 %

per-sheet scale spread (max-min)/median: 0.49%

HOLD-OUT  GCP 0: rms 0.97 mm max 1.82 mm (0.65%) ... GCP 5: rms 0.54 mm (0.36%)  the accuracy figure

floor plane through 24 corners: rms 2.5 mm, max 9.1 mm

tracker (x, y) seen from above is MIRRORED (det R = -1)                      expected, see 2.2

camera heights along the walk: median 1148 mm (range 696-1296)               plausible hand-held height

2       354      2250    -1.8                                              sheet centres within 2 mm of z = 0

dense cloud: 16,226,731 points, scale 840.63 mm/unit

inside the walk's hull: 14,500,017

floor refit: offset +4.5 mm, tilt 0.050 deg vs the sheet plane, floor rms 4.5 mm   two floors agree

wall top ~320 mm; band 30-340 mm: 1,966,858 points

DBSCAN: 8 clusters; outer 22650 cells, inner 10530 cells; dropped 6 (162 cells)

outer wall: length 15.572 m, encloses 17.231 m^2, points-to-line rms 28.1 mm (median 15.2)   = wall blur

inner wall: length 6.607 m, encloses 1.414 m^2, points-to-line rms 28.6 mm (median 15.9)

midline 10.858 m; width min 1123 / mean 1428 / max 1722 mm; drivable area 15.82 m^2

track A-1_B-1  : 1863 samples, outside corridor 0.00%, clearance min 154 / p5 331 / median 560 mm

track A-5b_B-5 : 3295 samples, outside corridor 0.00%, clearance min  33 / p5 269 / median 482 mm

Contribution 1 in numbers:

| Quantity | TrackMap_S2 | What it means |
| --- | --- | --- |
| Scale | 840.63 mm per SfM unit | median of six per-sheet scales |
| Per-sheet spread | 0.49 % | how well the six sheets agree with each other |
| Hold-out error | 0.54–0.97 mm RMS (0.36–0.65 %), worst length 1.82 mm | error on a 150 mm sheet that was not used for the scale |
| Corner triangulation | 0.73–1.24 px RMS, 164–303 inlier views | every one of the 24 corners |
| Sheet-corner plane | 2.5 mm RMS, 9.1 mm max | sheet flatness + triangulation together |
| Dense floor vs sheet plane | +4.5 mm offset, 0.05° tilt | two independent floor estimates |

Contribution 2 in numbers (all to wall centrelines):

| Quantity | TrackMap_S2 | Note |
| --- | --- | --- |
| Wall height | ~320 mm | from the height histogram |
| Outer wall | 15.57 m long, encloses 17.23 m² |  |
| Inner wall (island) | 6.61 m long, encloses 1.41 m² |  |
| Midline | 10.86 m | centring RMS 19.8 mm |
| Width | 1123 / 1428 / 1722 mm (min / mean / max) | centre to centre, not clear width |
| Drivable area | 15.82 m² |  |
| Wall points to line | 28 mm RMS, 15–16 mm median | wall blur in the cloud, not line error |
| Car tracks outside corridor | 0 % for all six runs (14,097 samples) | consistency, not accuracy |
| Minimum clearance | 154–432 mm (pairs 1–4), 33–38 mm (pairs 5, 5b) | see below |

Two numbers in this run were computed outside the scripts, by hand in the 28 September session, and are not reproduced by any file in the tree: the sheet centres from the map against each two-camera run's WorldFit.layout agree to 41–118 mm RMS, and the tracker's scale is 0.2–2.0 % larger than the map's. The second is the likely reason the pair 5 and 5b tracks pass within 33–38 mm of a wall centreline, which is less than half a car width; that explanation is unconfirmed.

What this run does not show. There is no absolute accuracy figure for the boundary. The session-2 track has been taken down and was never measured by hand, so the line can only be checked for consistency: hold-out on the sheets, the two floor estimates, the overlay, and the trajectories staying inside. The planned absolute checks are in 7.1.

# 6. Data structures and module dependencies at a glance

## 6.1 The objects that flow between modules

| Object | Made by | Consumed by | Holds |
| --- | --- | --- | --- |
| Detections | detector.detect_all_frames | floor.fit_floor, vehicle.locate_vehicle_aruco, fuse._tag_px, crossguide.guide (mutates) | corners per analysed frame, fps, size, counts |
| FloorFit | floor.fit_floor | vehicle.*, outputs.*, world.gcp_corners_mm, fuse._anchor_mm, crossguide, dual_outputs | static sheets, distortion, H, nadir, height, residuals |
| Track | vehicle.combine_and_smooth | outputs.*, fuse, crossguide, dual_outputs | smoothed and raw px/mm tracks, masks, path lengths |
| ClipResult | track_ground.process_clip | track_dual, fuse, crossguide, dual_outputs | det + floor + track + veh_px + t_global for one clip |
| Rigid2D | world.frame_from_layout, world.fit_world | everything two-camera | R, t; callable, invertible, composable |
| WorldFit | world.fit_world | track_dual, dual_outputs.write_topdown/overlay | shared ids, a_to_b, residuals, scale, a/b_to_world, layout |
| CamTrack | fuse._cam_on_grid | fuse._merge, fuse.kalman_smooth, dual_outputs | one camera on the grid: xy, tagpx, frame |
| Fused | fuse.fuse | fuse.print_fusion_summary, dual_outputs.* | grid, both CamTracks, merged xy, src, disagreement |
| <NAME>_metric.json | metric_scale.main | boundary.load_metric, boundary_overlay, run_boundary.sh | scale, R, t (SfM -> world mm), hold-out, sheet corners in world mm |
| Grid | boundary.main | equidistant_contour, the DBSCAN step, the corridor step | raster origin, cell size, shape; mm <-> (row, col) |
| <NAME>_boundary.csv / .json | boundary.main | boundary_overlay (csv), run_boundary.sh (wall_top_mm) | outer, inner, midline polylines in world mm; every reported number |

Pipeline B passes no Python objects between its stages: each stage is a separate process, and they communicate only through the files in output/<NAME>/. That is why metric_scale.py writes everything the later stages need into one JSON, and why a stage can be rerun on its own.

## 6.2 Who imports whom (pipeline A)

aruco_config  <- detector

gpu           <- track_ground, track_dual, calibrate_lens          (first import, sets env)

geometry      <- floor, vehicle, outputs, world, crossguide, dual_outputs, calibrate_lens

detector      <- track_ground, crossguide, calibrate_lens, metric_scale (pipeline B, via sys.path)

floor         <- vehicle, outputs, world, track_ground

vehicle       <- outputs, fuse (smooth_xy), track_ground

outputs       <- track_ground

timecode      <- track_ground, track_dual, dual_outputs

track_ground  <- track_dual, fuse (ClipResult), crossguide (ClipResult), dual_outputs (ClipResult)

world         <- track_dual, fuse (Rigid2D), crossguide (Rigid2D), dual_outputs (WorldFit)

fuse          <- track_dual, dual_outputs (Fused)

crossguide    <- track_dual

dual_outputs  <- track_dual

There are no circular imports: track_ground sits above the single-camera modules, and the two-camera modules sit above track_ground. The one link between the pipelines is metric_scale.py importing detector.build_detector (and through it aruco_config); boundary.py and boundary_overlay.py import nothing from the tree.

## 6.3 Every file a run produces

| Run | File | What it is |
| --- | --- | --- |
| run_tracking.sh X | output/X/X_trajectory.csv | per analysed frame; see 3.8 |
|  | output/X/X_overlay.mp4 | the clip with GCP outlines, the track and the current fix drawn |
|  | output/X/X.log | everything printed, incl. the floor fit report |
| run_dual.sh A B | output/A_B/A_B_fused.csv | per grid sample; see 4.6 |
|  | output/A_B/A_B_topdown.png | world-frame plot of both cameras' fixes, the fused track and the sheets |
|  | output/A_B/A_B_overlay.mp4 | side-by-side overlay, fused track reprojected into both views |
|  | output/A_B/A/, output/A_B/B/ | the two single-camera output sets (CSV + overlay), after cross-guidance |
|  | output/A_B/A_B.log | both floor fits, world fit, guidance, tag-height solve, fusion summary |
| run_mapping.sh … | output/N/sparse/ | the COLMAP model (cameras/images/points3D.bin), largest component |
|  | output/N/N_sparse.ply, N_dense.ply | sparse and dense clouds, SfM units; open in CloudCompare, Colors → RGB |
|  | output/N/N_topdown.png | sparse cloud on the RANSAC floor plane with the camera path |
|  | output/N/sources.txt, N.log | which clips built it; everything printed |
| run_splat.sh N d | output/N/splat_d<d>/N_splat_d<d>.ply | Gaussian splat; view in SuperSplat, not a point-cloud viewer |
| run_boundary.sh N | output/N/N_metric.json, _metric.log | scale, world transform, hold-out; see 5.5 |
|  | output/N/N_boundary.csv, .dxf, .json | wall centrelines + midline (world mm), CAD file, all numbers |
|  | output/N/N_boundary_topdown.png, _width.png | metric top-down with car tracks; width along the midline |
|  | output/N/N_walls3d.ply | the two walls extruded to the measured height |
|  | output/N/N_boundary_overlay.mp4, .jpg | boundary projected into the walk; open this first |
|  | output/N/N_boundary.log, N.log | boundary printout; the runner log (appended) |

# 7. What the code does not do yet, and what to be careful of

## 7.1 Not in git_clone/

- Metric scale and boundary — built for TrackMap_S2 on the COLMAP cloud only (28 September). Still to do: run them on the GLUEMAP cloud of the same walk (sparse and dense exist in Map_Test/output/gluemap_TrackMap_S2/; it gives crisper barrier lines and a different SfM scale, so it is a real second test); run them on the session-1 captures; and the walk-1 against walk-2 repeatability test (reconstruct each walk alone and compare the two boundaries).

- An absolute accuracy figure for the boundary. Nothing in the tree can produce one without a measured reference. Options: the session-1 captures against hand measurements from that session (to be checked: what was measured, and whether that track was tape or walls); a surveyed GCP set in a new capture; or a few tape measurements of wall-to-wall width on a track that is still standing. A --survey hook in metric_scale.py (fit to surveyed sheet positions with a 3-D Umeyama instead of side lengths) is the natural extension.

- Plots in the single-camera run (outputs.write_plots) are disabled pending the restyle.

- Porting to GIT/. None of the 21 September work (floor.py fix, 175 mm default, two-camera pipeline, calibrate_lens) and none of the 28 September work (run_boundary.sh and its three scripts) is in the git repository yet. That is Antwan's call.

- ChArUco calibration, GCP survey, camera-height measurement, tag-height measurement, onboard logs for ATE/RPE — all need another capture.

- Calibrate-then-lift. The supervisor's rule (29 September) is that no sheets may lie on the track while the car drives. The current tracker finds the sheets in the driving clip itself, so on footage captured under the new rule fit_floor would find no static sheets and stop. track_ground.py / track_dual.py must be changed to take the floor fit, lens and pose from a separate calibration clip of the same camera position. The mapping side is unaffected (the sheets may be down during the walk).

## 7.2 Things that will bite

- Three copies of the tracking code. A fix in git_clone/ is not in GIT/ or Tracking/src/. The Mono1–3 numbers in Tracking/output/ are from the old anchor rule and should not be quoted.

- A-N/B-N is the pair number, not the camera position. A-4 and A-5 are at position A2; B-5 is at B2.

- --tag-heights 20:175 is a lookup, not a measurement. Every session-2 run before 21 September used 57 mm.

- Camera A's fits are poor at every position (k1 at the bound, 30 mm sheet residual, 6 % scale disagreement with B). Do not read its recovered height as the camera height. Check its lens mode (serial C3531325316303) and shoot ChArUco on both bodies in the same mode.

- The world fit needs GCP 1 and GCP 5 accepted by at least one camera, and at least 2 sheets shared. --world-origin / --world-x change the definition; the mapping side must then use the same pair.

- Cross-guidance and the windowed search each add a second decode pass over a clip; --no-cross-guide / --no-window-search skip them when iterating on something else.

- Stop every running job before editing any file a job might read (pgrep -f 'python track_dual.py'). Python imports modules at start, but the runners cd src/ and a batch launched later picks up whatever is on disk.

- The single-camera overlay still draws one polyline through all located frames (a chord across dropouts); the two-camera overlay and top-down split at gaps. If the mono overlay is ever restyled, do the same there.

- aruco_config.py carries old constants (A4_SHEET_SIDE_MM etc.) that nothing uses; the sheet side is --gcp-mm.

- The world frame is mirrored seen from above (2.2, 5.5), in both pipelines on purpose. Every top-down PNG is a mirror image of the real track. Flip both pipelines together or neither.

- "Fit rms 28 mm" is not accuracy. It is in the boundary top-down title and the JSON; it measures how thick the wall is in the cloud. The accuracy evidence is the hold-out (0.36–0.65 %) for scale and, for the boundary, only consistency checks so far.

- requirements.txt lacks scikit-learn, which boundary.py imports for DBSCAN. It is installed in the working .venv (1.9.0), so nothing fails here, but a fresh clone would fail at the boundary step. Add scikit-learn to the file when porting.

- run_boundary.sh can miss a failure. Its || { say FAILED; continue; } follows a pipe into tee, so it never fires; the real check is the [ -f … ] test on the output file after it. A stale _metric.json or _boundary.csv from an earlier run passes that test. After a failed stage, delete the old output before rerunning, or read the log.

- boundary.py expects exactly one island. It stops unless it finds a closed outer wall with one closed loop inside it. A track with no island, two islands, or an outer wall with a gap wider than the 60 mm closing needs a code change, not a flag.

- The dense PLY in git_clone/…/TrackMap_S2/ is a hard link to Mapping/output/TrackMap_S2/. Overwriting it in place in either folder changes both; deleting one leaves the other.

- The corridor check reads every fused CSV in pipeline_A_tracking/output/. That is right for session 2 only; for another track set TRACKS= to the runs that belong to it, or the check is meaningless.

## 7.3 Other files in the tree

- README.md — the user-facing description of both pipelines: requirements, marker roles, how to run, what each output is, the settings and why, typical timings. Written for someone cloning the repo, so it covers neither the two-camera code (it says "not in this repo": true for GIT/, out of date for git_clone/) nor run_boundary.sh, which it still lists as future work.

- PLAN_two_camera.md — the two-camera plan as approved (data probe, approach, file-by-file change list, checkpoints, decisions) followed by the 'What was built and found' record with the five findings of 21 September.

- requirements.txt — pycolmap==4.1.1, opencv-contrib-python==5.0.0.93, numpy==2.5.1, scipy==1.18.0, open3d==0.19.0, matplotlib==3.11.1. The .venv is at the repo root (git_clone/.venv or the working tree's).

- .gitignore — keeps capture/ and output/ folders but not their contents; ignores all media, .ply, .png, logs, databases, *_work/, __pycache__, .venv.

- pipeline_A_tracking/calib/*.log — the console output of the three calibrate_lens.py runs.